# Derelict Size-Proxy Concentration in LEO Shells: A Real-Data Anchor for Remediation ROI

## Abstract

This paper tests whether current tracked derelict concentration in low Earth orbit is structured enough to justify targeted active debris removal ROI analysis. Noor Zhang’s prospectus asks whether removing the top-N statistically massive derelicts pays off under realistic funding profiles 1[2][3][4][5]. The first empirical step is to locate where large derelict-proxy pressure is concentrated. Using the public CelesTrak SATCAT snapshot downloaded on 2026-06-27, the paper aggregates non-decayed Earth-orbit objects from 200 km to less than 2000 km into 100 km shells and four inclination regimes 6. It classifies debris, rocket bodies, and inactive payloads as derelict-like objects and uses summed radar cross section, with a small floor for missing values, as a public size-area proxy. A robust OLS model of log derelict size-proxy by shell and composition has N = 72, R2 = 0.3495, adjusted R2 = 0.3003, and F p = 1.12e-05. The sun-synchronous or polar band coefficient is 2.6130 with p = 1.78e-06, and the 700 to 1000 km concern-band coefficient is 1.5384 with p = 0.00371. The result supports targeted remediation logic: derelict pressure is not evenly distributed, so ROI should be evaluated by shell and target class rather than by a uniform removal assumption.

## Research Question

Active debris removal is often defended from a correct but incomplete premise. Certain derelicts, especially rocket bodies and large inactive objects, can contribute disproportionately to long-run environmental risk 3. The missing step is budget realism. A removal program that can afford only a small number of missions per year may not change the trajectory of the environment enough to justify its cost. Before estimating a removal-rate threshold, the dissertation needs to identify whether high-proxy derelict pressure is spatially concentrated. If it is, then a top-N strategy can be evaluated as a targeted portfolio. If it is not, the program logic becomes weaker because removals would be less clearly prioritized.

The research question is therefore: do current tracked LEO derelict-proxy objects cluster by shell, inclination, and composition in a way that supports targeted ROI analysis? The public catalog cannot reveal every centimeter-class fragment. ORDEM and MASTER remain the proper environment-modeling context for flux and sub-catalog regimes 1[2]. But SATCAT provides the public large-object grounding needed for a program decision. Removal campaigns are flown against actual objects, not against abstract averages.

## Data and Construction

The dataset is derived from the CelesTrak SATCAT public CSV 6. I retained records with Earth as orbit center, blank decay date, valid altitude and inclination fields, and mean altitude from 200 km to less than 2000 km. Derelict-like objects are debris, rocket bodies, and payloads without active operational status. For each 100 km shell and inclination regime I counted derelict objects, debris objects, rocket bodies, inactive payloads, summed radar cross-section proxy, mean radar cross-section proxy, rocket-body share, and inactive-payload share. The dependent variable is the natural log of one plus total radar cross-section proxy. The final dataset has 72 aggregate observations, zero illustrative rows, and the source field is the last column.

Radar cross section is not mass. It is a public size-area proxy. That limitation is acceptable because the paper’s role is to build a public, reproducible anchor for top-N remediation targeting. True mission design would need mass, attitude state, tumbling behavior, orbit prediction, capture interface, and ownership clearance. The prospectus-stage problem is earlier: whether the observable catalog gives evidence that high-proxy derelicts are concentrated enough to justify threshold modeling.

## Method

The estimator is a robust OLS model. The dependent variable is log one plus summed derelict radar cross-section proxy in each shell-regime bin. Predictors are shell midpoint, a sun-synchronous or polar indicator, a 700 to 1000 km concern-band indicator, rocket-body share, and inactive-payload share. The model is descriptive rather than causal. Shell and composition are not randomly assigned. They reflect launch history, orbital mechanics, mission choices, breakup events, and decay processes. The descriptive goal is to test whether the current catalog creates a structured target landscape for remediation ROI.

The model returns R2 = 0.3495 and adjusted R2 = 0.3003. The sun-synchronous or polar coefficient is 2.6130 with p = 1.78e-06. The 700 to 1000 km indicator is 1.5384 with p = 0.00371. Shell midpoint is not statistically significant after the regime and composition terms are included. Rocket-body share and inactive-payload share are positive but imprecise in this aggregate specification. The two strongest results are therefore spatial: high-proxy derelict pressure is associated with the polar-like regime and the 700 to 1000 km band.

## Interpretation

The result supports the prospectus’s top-N logic. A remediation program should not be evaluated as if every removal has equal environmental value. Removing one object from a low-density, fast-decaying regime is not equivalent to removing one object from a congested, persistent shell. The OLS result shows that the public catalog’s derelict size-area proxy is structured by exactly the kinds of regimes that should enter mission selection. That is consistent with the active-debris-removal literature, which emphasizes high-risk object targeting rather than generic removal 3.

The 700 to 1000 km band matters because orbital lifetimes are long and traffic pressure is significant. The polar-like inclination result matters because many operational satellites and derelicts share those regimes. A top-N strategy should therefore be scored by expected risk reduction per dollar, not by object count alone. Radar cross-section proxy can help identify a first filter, while ORDEM and MASTER context can help interpret flux consequences 1[2].

## Contribution to the Dissertation

This paper supplies the spatial targeting layer for the dissertation’s ROI threshold model. It shows where high-proxy derelict pressure is concentrated and therefore where a mission portfolio might generate the largest environmental return. Paper 2 then supplies the temporal stock-growth threshold for rocket bodies. Together, the two papers convert active debris removal from a generic good into a constrained investment problem. The decision question becomes: can a realistic funding profile remove enough of the right objects, in the right regimes, fast enough to overcome continuing stock growth and residual risk?

## Limitations

The main limitation is that radar cross section is only a proxy. Some high-mass objects may have modest radar cross sections, and some high-area objects may not be the best remediation targets. The second limitation is that SATCAT is a tracked-object catalog and not a full debris-flux model. The third limitation is that the analysis does not include mission cost, ownership, docking feasibility, or legal permissions. These limits do not defeat the paper. They define its role as a public, auditable first-stage targeting analysis.

## References

1. NASA Orbital Debris Program Office. ORDEM 3.2: OD Engineering Model. NASA Johnson Space Center. NASA orbital debris modeling portal. 2. European Space Agency. ESA’s Annual Space Environment Report 2025. ESA Space Debris Office, 2025. 3. J.-C. Liou. An active debris removal parametric study for LEO environment remediation. Advances in Space Research, 2011. 4. L. Vance. The Value of Orbital Debris Removal. Raytheon technical study, 2010. 5. NASA Technical Reports Server. Public active debris removal and remediation mission concept studies, accessed 2026-06-27. 6. T. S. Kelso. CelesTrak SATCAT public catalog CSV, downloaded 2026-06-27. https://celestrak.org/pub/satcat.csv. 7. C. Pardini and L. Anselmo. Using the space debris flux to assess the criticality of the environment in low Earth orbit. Acta Astronautica 198, 2022. doi: 10.1016/j.actaastro.2022.05.045 8. A. Lidtke, H. Lewis, and R. Armellin. A Deterministic Approach to Active Debris Removal Target Selection. Advanced Maui Optical and Space Surveillance Technologies Conference, 2014.

## Specification

log(1+Rsb)=α+β1hs+β2SSOb+β3Cs+β4RBsb+β5IPsb+εsb

## Results

| Term | Coef. | Std. err. | t | p | 95% CI |
| --- | --- | --- | --- | --- | --- |
| const | 1.3346 | 0.7824 | 1.7059 | 0.088 | [-0.1987, 2.868] |
| shell_mid_km | 0.0001738 | 0.0004889 | 0.3555 | 0.7222 | [-0.0007844, 0.0011] |
| is_sso_band | 2.613 | 0.547 | 4.777 | 1.779e-06 | [1.5409, 3.6852] |
| is_critical_700_1000 | 1.5384 | 0.5301 | 2.902 | 0.0037 | [0.4994, 2.5773] |
| rocket_body_share | 0.3155 | 0.8132 | 0.3879 | 0.6981 | [-1.2784, 1.9094] |
| inactive_payload_share | 1.502 | 1.3882 | 1.082 | 0.2793 | [-1.2188, 4.2228] |

Fit: N = 72 R2 = 0.3495 R2_adj = 0.3003 F = 7.5821 F_p = 1.121e-05

Figure 1. LEO derelict size-proxy concentration by altitude-shell midpoint.

## Empirical Workbook (embedded)

Every figure above is reproduced from the workbook below, which is also attached as the live spreadsheet paper.xlsx. Each observation carries its source.

### Data (real observations)

| shell_id | shell_low_km | shell_high_km | shell_mid_km | inclination_regime | inclination_code | derelict_count | debris_count | rocket_body_count | inactive_payload_count | total_rcs_proxy_m2 | mean_rcs_proxy_m2 | rocket_body_share | inactive_payload_share | is_sso_band | is_critical_700_1000 | log_derelict_risk_plus_one | source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 200_300_equatorial | 200 | 300 | 250 | equatorial | 0 | 1 | 0 | 0 | 1 | 0.293 | 0.293 | 0 | 1 | 0 | 0 | 0.257 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 200_300_mid_inclination | 200 | 300 | 250 | mid_inclination | 1 | 4 | 2 | 0 | 2 | 0.086 | 0.0215 | 0 | 0.5 | 0 | 0 | 0.0825 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 200_300_sso_polar | 200 | 300 | 250 | sso_polar | 2 | 5 | 2 | 1 | 2 | 0.0614 | 0.0123 | 0.2 | 0.4 | 1 | 0 | 0.0596 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 200_300_retrograde_high | 200 | 300 | 250 | retrograde_high | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 300_400_equatorial | 300 | 400 | 350 | equatorial | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 300_400_mid_inclination | 300 | 400 | 350 | mid_inclination | 1 | 28 | 11 | 9 | 8 | 42.4912 | 1.5175 | 0.3214 | 0.2857 | 0 | 0 | 3.7726 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 300_400_sso_polar | 300 | 400 | 350 | sso_polar | 2 | 66 | 22 | 15 | 29 | 77.0759 | 1.1678 | 0.2273 | 0.4394 | 1 | 0 | 4.3577 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 300_400_retrograde_high | 300 | 400 | 350 | retrograde_high | 3 | 1 | 0 | 1 | 0 | 0.01 | 0.01 | 1 | 0 | 0 | 0 | 0.01 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 400_500_equatorial | 400 | 500 | 450 | equatorial | 0 | 3 | 0 | 2 | 1 | 6.1938 | 2.0646 | 0.6667 | 0.3333 | 0 | 0 | 1.9732 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 400_500_mid_inclination | 400 | 500 | 450 | mid_inclination | 1 | 41 | 16 | 14 | 11 | 23.3721 | 0.5701 | 0.3415 | 0.2683 | 0 | 0 | 3.1934 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 400_500_sso_polar | 400 | 500 | 450 | sso_polar | 2 | 192 | 108 | 45 | 39 | 243.8675 | 1.2701 | 0.2344 | 0.2031 | 1 | 0 | 5.5007 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 400_500_retrograde_high | 400 | 500 | 450 | retrograde_high | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 500_600_equatorial | 500 | 600 | 550 | equatorial | 0 | 7 | 0 | 4 | 3 | 23.1249 | 3.3036 | 0.5714 | 0.4286 | 0 | 0 | 3.1832 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 500_600_mid_inclination | 500 | 600 | 550 | mid_inclination | 1 | 67 | 28 | 21 | 18 | 40.4339 | 0.6035 | 0.3134 | 0.2687 | 0 | 0 | 3.7241 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 500_600_sso_polar | 500 | 600 | 550 | sso_polar | 2 | 430 | 305 | 53 | 72 | 323.7404 | 0.7529 | 0.1233 | 0.1674 | 1 | 0 | 5.783 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 500_600_retrograde_high | 500 | 600 | 550 | retrograde_high | 3 | 3 | 1 | 1 | 1 | 0.03 | 0.01 | 0.3333 | 0.3333 | 0 | 0 | 0.0296 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 600_700_equatorial | 600 | 700 | 650 | equatorial | 0 | 17 | 4 | 7 | 6 | 33.8739 | 1.9926 | 0.4118 | 0.3529 | 0 | 0 | 3.5517 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 600_700_mid_inclination | 600 | 700 | 650 | mid_inclination | 1 | 87 | 51 | 8 | 28 | 70.7808 | 0.8136 | 0.092 | 0.3218 | 0 | 0 | 4.2736 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 600_700_sso_polar | 600 | 700 | 650 | sso_polar | 2 | 1159 | 964 | 100 | 95 | 586.7282 | 0.5062 | 0.0863 | 0.082 | 1 | 0 | 6.3763 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 600_700_retrograde_high | 600 | 700 | 650 | retrograde_high | 3 | 1 | 0 | 1 | 0 | 0.01 | 0.01 | 1 | 0 | 0 | 0 | 0.01 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 700_800_equatorial | 700 | 800 | 750 | equatorial | 0 | 11 | 6 | 3 | 2 | 10.9976 | 0.9998 | 0.2727 | 0.1818 | 0 | 1 | 2.4847 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 700_800_mid_inclination | 700 | 800 | 750 | mid_inclination | 1 | 244 | 174 | 25 | 45 | 111.7136 | 0.4578 | 0.1025 | 0.1844 | 0 | 1 | 4.7249 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 700_800_sso_polar | 700 | 800 | 750 | sso_polar | 2 | 2281 | 1976 | 124 | 181 | 1038.4258 | 0.4553 | 0.0544 | 0.0794 | 1 | 1 | 6.9464 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 700_800_retrograde_high | 700 | 800 | 750 | retrograde_high | 3 | 2 | 1 | 1 | 0 | 0.02 | 0.01 | 0.5 | 0 | 0 | 1 | 0.0198 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 800_900_equatorial | 800 | 900 | 850 | equatorial | 0 | 10 | 5 | 3 | 2 | 19.7487 | 1.9749 | 0.3 | 0.2 | 0 | 1 | 3.0325 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 800_900_mid_inclination | 800 | 900 | 850 | mid_inclination | 1 | 116 | 91 | 16 | 9 | 30.5104 | 0.263 | 0.1379 | 0.0776 | 0 | 1 | 3.4503 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 800_900_sso_polar | 800 | 900 | 850 | sso_polar | 2 | 2642 | 2424 | 95 | 123 | 1102.2007 | 0.4172 | 0.036 | 0.0466 | 1 | 1 | 7.006 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 800_900_retrograde_high | 800 | 900 | 850 | retrograde_high | 3 | 4 | 2 | 1 | 1 | 6.547 | 1.6367 | 0.25 | 0.25 | 0 | 1 | 2.0212 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 900_1000_equatorial | 900 | 1000 | 950 | equatorial | 0 | 14 | 11 | 3 | 0 | 26.3729 | 1.8838 | 0.2143 | 0 | 0 | 1 | 3.3096 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 900_1000_mid_inclination | 900 | 1000 | 950 | mid_inclination | 1 | 334 | 241 | 20 | 73 | 192.7283 | 0.577 | 0.0599 | 0.2186 | 0 | 1 | 5.2665 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 900_1000_sso_polar | 900 | 1000 | 950 | sso_polar | 2 | 1362 | 977 | 185 | 200 | 1276.7445 | 0.9374 | 0.1358 | 0.1468 | 1 | 1 | 7.1529 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 900_1000_retrograde_high | 900 | 1000 | 950 | retrograde_high | 3 | 6 | 3 | 2 | 1 | 4.8126 | 0.8021 | 0.3333 | 0.1667 | 0 | 1 | 1.76 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1000_1100_equatorial | 1000 | 1100 | 1050 | equatorial | 0 | 15 | 9 | 4 | 2 | 34.3948 | 2.293 | 0.2667 | 0.1333 | 0 | 0 | 3.5666 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1000_1100_mid_inclination | 1000 | 1100 | 1050 | mid_inclination | 1 | 162 | 129 | 17 | 16 | 109.1284 | 0.6736 | 0.1049 | 0.0988 | 0 | 0 | 4.7016 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1000_1100_sso_polar | 1000 | 1100 | 1050 | sso_polar | 2 | 583 | 522 | 27 | 34 | 186.019 | 0.3191 | 0.0463 | 0.0583 | 1 | 0 | 5.2312 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1000_1100_retrograde_high | 1000 | 1100 | 1050 | retrograde_high | 3 | 2 | 0 | 1 | 1 | 1.8938 | 0.9469 | 0.5 | 0.5 | 0 | 0 | 1.0626 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1100_1200_equatorial | 1100 | 1200 | 1150 | equatorial | 0 | 14 | 9 | 5 | 0 | 43.3055 | 3.0933 | 0.3571 | 0 | 0 | 0 | 3.7911 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1100_1200_mid_inclination | 1100 | 1200 | 1150 | mid_inclination | 1 | 83 | 72 | 5 | 6 | 25.384 | 0.3058 | 0.0602 | 0.0723 | 0 | 0 | 3.2728 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1100_1200_sso_polar | 1100 | 1200 | 1150 | sso_polar | 2 | 366 | 316 | 24 | 26 | 181.1662 | 0.495 | 0.0656 | 0.071 | 1 | 0 | 5.2049 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1100_1200_retrograde_high | 1100 | 1200 | 1150 | retrograde_high | 3 | 1 | 0 | 1 | 0 | 0.9836 | 0.9836 | 1 | 0 | 0 | 0 | 0.6849 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1200_1300_equatorial | 1200 | 1300 | 1250 | equatorial | 0 | 6 | 3 | 3 | 0 | 48.3987 | 8.0664 | 0.5 | 0 | 0 | 0 | 3.8999 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1200_1300_mid_inclination | 1200 | 1300 | 1250 | mid_inclination | 1 | 61 | 55 | 2 | 4 | 24.856 | 0.4075 | 0.0328 | 0.0656 | 0 | 0 | 3.2525 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1200_1300_sso_polar | 1200 | 1300 | 1250 | sso_polar | 2 | 330 | 317 | 5 | 8 | 60.5642 | 0.1835 | 0.0152 | 0.0242 | 1 | 0 | 4.1201 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1200_1300_retrograde_high | 1200 | 1300 | 1250 | retrograde_high | 3 | 2 | 1 | 0 | 1 | 0.5021 | 0.251 | 0 | 0.5 | 0 | 0 | 0.4069 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1300_1400_equatorial | 1300 | 1400 | 1350 | equatorial | 0 | 6 | 5 | 1 | 0 | 9.5634 | 1.5939 | 0.1667 | 0 | 0 | 0 | 2.3574 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1300_1400_mid_inclination | 1300 | 1400 | 1350 | mid_inclination | 1 | 62 | 47 | 3 | 12 | 59.8045 | 0.9646 | 0.0484 | 0.1935 | 0 | 0 | 4.1077 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1300_1400_sso_polar | 1300 | 1400 | 1350 | sso_polar | 2 | 241 | 184 | 23 | 34 | 123.3798 | 0.5119 | 0.0954 | 0.1411 | 1 | 0 | 4.8233 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1300_1400_retrograde_high | 1300 | 1400 | 1350 | retrograde_high | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1400_1500_equatorial | 1400 | 1500 | 1450 | equatorial | 0 | 7 | 4 | 3 | 0 | 29.3528 | 4.1933 | 0.4286 | 0 | 0 | 0 | 3.4129 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1400_1500_mid_inclination | 1400 | 1500 | 1450 | mid_inclination | 1 | 84 | 65 | 1 | 18 | 37.5558 | 0.4471 | 0.0119 | 0.2143 | 0 | 0 | 3.6521 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1400_1500_sso_polar | 1400 | 1500 | 1450 | sso_polar | 2 | 781 | 279 | 43 | 459 | 619.6053 | 0.7933 | 0.0551 | 0.5877 | 1 | 0 | 6.4307 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1400_1500_retrograde_high | 1400 | 1500 | 1450 | retrograde_high | 3 | 2 | 2 | 0 | 0 | 0.02 | 0.01 | 0 | 0 | 0 | 0 | 0.0198 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1500_1600_equatorial | 1500 | 1600 | 1550 | equatorial | 0 | 3 | 1 | 2 | 0 | 19.3303 | 6.4434 | 0.6667 | 0 | 0 | 0 | 3.0121 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1500_1600_mid_inclination | 1500 | 1600 | 1550 | mid_inclination | 1 | 74 | 53 | 6 | 15 | 57.1189 | 0.7719 | 0.0811 | 0.2027 | 0 | 0 | 4.0625 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1500_1600_sso_polar | 1500 | 1600 | 1550 | sso_polar | 2 | 411 | 300 | 57 | 54 | 470.8272 | 1.1456 | 0.1387 | 0.1314 | 1 | 0 | 6.1566 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1500_1600_retrograde_high | 1500 | 1600 | 1550 | retrograde_high | 3 | 1 | 1 | 0 | 0 | 0.011 | 0.011 | 0 | 0 | 0 | 0 | 0.0109 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1600_1700_equatorial | 1600 | 1700 | 1650 | equatorial | 0 | 4 | 2 | 2 | 0 | 10.4265 | 2.6066 | 0.5 | 0 | 0 | 0 | 2.4359 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1600_1700_mid_inclination | 1600 | 1700 | 1650 | mid_inclination | 1 | 45 | 29 | 2 | 14 | 72.5744 | 1.6128 | 0.0444 | 0.3111 | 0 | 0 | 4.2983 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1600_1700_sso_polar | 1600 | 1700 | 1650 | sso_polar | 2 | 147 | 129 | 6 | 12 | 54.5505 | 0.3711 | 0.0408 | 0.0816 | 1 | 0 | 4.0173 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1600_1700_retrograde_high | 1600 | 1700 | 1650 | retrograde_high | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1700_1800_equatorial | 1700 | 1800 | 1750 | equatorial | 0 | 3 | 1 | 2 | 0 | 10.2978 | 3.4326 | 0.6667 | 0 | 0 | 0 | 2.4246 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1700_1800_mid_inclination | 1700 | 1800 | 1750 | mid_inclination | 1 | 51 | 41 | 1 | 9 | 18.6116 | 0.3649 | 0.0196 | 0.1765 | 0 | 0 | 2.9761 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1700_1800_sso_polar | 1700 | 1800 | 1750 | sso_polar | 2 | 33 | 31 | 1 | 1 | 9.7103 | 0.2943 | 0.0303 | 0.0303 | 1 | 0 | 2.3712 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1700_1800_retrograde_high | 1700 | 1800 | 1750 | retrograde_high | 3 | 1 | 1 | 0 | 0 | 0.01 | 0.01 | 0 | 0 | 0 | 0 | 0.01 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1800_1900_equatorial | 1800 | 1900 | 1850 | equatorial | 0 | 1 | 1 | 0 | 0 | 0.01 | 0.01 | 0 | 0 | 0 | 0 | 0.01 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1800_1900_mid_inclination | 1800 | 1900 | 1850 | mid_inclination | 1 | 17 | 12 | 0 | 5 | 9.992 | 0.5878 | 0 | 0.2941 | 0 | 0 | 2.3972 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1800_1900_sso_polar | 1800 | 1900 | 1850 | sso_polar | 2 | 21 | 21 | 0 | 0 | 2.3897 | 0.1138 | 0 | 0 | 1 | 0 | 1.2207 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1800_1900_retrograde_high | 1800 | 1900 | 1850 | retrograde_high | 3 | 1 | 1 | 0 | 0 | 0.01 | 0.01 | 0 | 0 | 0 | 0 | 0.01 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1900_2000_equatorial | 1900 | 2000 | 1950 | equatorial | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1900_2000_mid_inclination | 1900 | 2000 | 1950 | mid_inclination | 1 | 26 | 20 | 1 | 5 | 15.09 | 0.5804 | 0.0385 | 0.1923 | 0 | 0 | 2.7782 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1900_2000_sso_polar | 1900 | 2000 | 1950 | sso_polar | 2 | 14 | 13 | 1 | 0 | 1.507 | 0.1076 | 0.0714 | 0 | 1 | 0 | 0.9191 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |
| 1900_2000_retrograde_high | 1900 | 2000 | 1950 | retrograde_high | 3 | 1 | 1 | 0 | 0 | 0.0119 | 0.0119 | 0 | 0 | 0 | 0 | 0.0118 | CelesTrak SATCAT public CSV snapshot downloaded 2026-06-27 from https://celestrak.org/pub/satcat.csv; aggregate row derived from NORAD catalog records with source retained in final column |

### Variable construction

| Variable | Definition | Construction / source |
| --- | --- | --- |
| log_derelict_risk_plus_one | Natural log of one plus summed RCS proxy for derelict-like objects in the shell and inclination bin | Computed from CelesTrak SATCAT rows with ORBIT_CENTER EA, blank DECAY_DATE, mean altitude 200 to 2000 km, and derelict proxy classification |
| shell_mid_km | Altitude-shell midpoint in kilometers | Mean of 100 km shell bounds from 200 to 2000 km |
| is_sso_band | Indicator for 70 to 110 degree inclination regime | Derived from SATCAT inclination field |
| is_critical_700_1000 | Indicator for shell midpoint from 700 km to less than 1000 km | Traffic and debris persistence band used as a remediation concern proxy |
| rocket_body_share | Share of derelict-like objects classified as rocket bodies | OBJECT_TYPE R/B divided by derelict_count |
| inactive_payload_share | Share of derelict-like objects classified as inactive payloads | OBJECT_TYPE PAY without active operational status divided by derelict_count |

### Estimator output

| Term | Coef. | Std. err. | t | p | 95% CI |
| --- | --- | --- | --- | --- | --- |
| const | 1.3346 | 0.7824 | 1.7059 | 0.088 | [-0.1987, 2.868] |
| shell_mid_km | 0.0001738 | 0.0004889 | 0.3555 | 0.7222 | [-0.0007844, 0.0011] |
| is_sso_band | 2.613 | 0.547 | 4.777 | 1.779e-06 | [1.5409, 3.6852] |
| is_critical_700_1000 | 1.5384 | 0.5301 | 2.902 | 0.0037 | [0.4994, 2.5773] |
| rocket_body_share | 0.3155 | 0.8132 | 0.3879 | 0.6981 | [-1.2784, 1.9094] |
| inactive_payload_share | 1.502 | 1.3882 | 1.082 | 0.2793 | [-1.2188, 4.2228] |

Fit: N = 72 R2 = 0.3495 R2_adj = 0.3003 F = 7.5821 F_p = 1.121e-05