# Spectrum Versus Photons: An OLS Cost Model of Space Communications Terminals Controlling for Throughput, Mass, and Power

## 1. Introduction

Space communications planners are asked, with rising frequency, whether free-space optical terminals deliver capacity at a lower recurring cost than the radio-frequency terminals they have flown for sixty years. The companion paper to this one fits a single bivariate log-log curve through a pooled sample of terminals and reads a cost-capacity elasticity off the slope. That specification is informative but undercontrolled: terminals differ in mass and power as well as in throughput and modality, and the cost engineer who signs a budget exhibit knows that mass and power often drive recurring cost more than peak data rate does. This paper therefore re-asks the same question with a multivariate ordinary least squares (OLS) model that conditions on terminal mass and power and uses a modality dummy to test whether the photon channel sits on a structurally lower cost surface once the engineering envelope is held fixed. The contribution is a partial coefficient on the optical indicator that the bivariate curve cannot recover.

## 2. Theory and Hypotheses

The cost-engineering literature on space hardware [1, 2] has long modelled recurring cost as a multiplicative function of mass, power, and a small set of complexity drivers, which justifies an additive log specification. The communications-engineering literature [3, 4] treats throughput as a function of aperture, transmit power, and link geometry, with optical terminals offering a higher Shannon ceiling per kilogram of aperture and per watt of DC input than radio-frequency terminals at comparable mission distance 5. If those engineering claims translate into procurement budgets, then after the model absorbs mass, power, and throughput, the residual modality indicator should carry a negative sign: optical wins on the same engineering envelope. The space-economics literature [6, 7] is less settled and admits the rival hypothesis that optical terminals carry a non-recurring engineering and qualification premium that persists in unit price into the early operational decade.

H1 (throughput sub-linearity). Conditional on mass and power, the elasticity of unit cost with respect to peak data rate is positive but less than unity, so doubling throughput raises unit cost by less than a factor of two.

H2 (modality discount). After conditioning on the throughput, mass, and power of a terminal, the coefficient on the optical indicator is negative and statistically distinguishable from zero.

## 3. Data

The sample is a cross-section of thirty-two space-communications terminals flown or contracted between 1995 and 2024, covering NASA, ESA, USSF, USAF, and commercial missions and spanning fourteen optical and eighteen radio-frequency systems. Each row records peak data rate in gigabits per second, terminal dry mass in kilograms, payload steady-state DC power in watts, and recurring unit cost in 2024 USD millions, together with a citation field identifying the primary source. Cost figures originate from GAO Major-Project Assessments (GAO-15-274, GAO-15-342, GAO-19-417, GAO-20-439, GAO-22-105212, GAO-22-105230), NASA Office of Inspector General audits, Department of the Air Force FY24 budget exhibits, SEC 10-K filings of Maxar, Inmarsat, Viasat, SES, Iridium, and Telesat, and contractor disclosures from Boeing, Thales Alenia, MDA, Mynaric, and Tesat. Throughput figures originate from NASA NTRS technical papers (including LCRD, TBIRD, LLCD, OPALS, ILLUMA-T, and DSOC reports), FCC International Bureau filings (SAT-MOD-20200417 for Starlink optical inter-satellite links), ESA EDRS programme documentation, Acta Astronautica refereed papers on NICT HICALI and CubeLCT, and vendor data sheets for the Tesat CONDOR and Mynaric CONDOR Mk3 lines. A small number of cost-share allocations are flagged in the source field as illustrative where the prime contractor reports only a programme-level figure and not a comms-subsystem subtotal; these comprise fewer than fifteen percent of rows.

## 4. Method

The estimator is ordinary least squares with HC1 heteroskedasticity-consistent standard errors on a log-log specification of unit cost as a function of peak data rate, terminal mass, payload power, and an optical modality indicator. The log transform on the continuous regressors lets the coefficients on data rate, mass, and power be read as constant elasticities. The optical dummy absorbs the average difference in log unit cost between an optical and a radio-frequency terminal of the same throughput, mass, and power. Robust standard errors are used because the sample mixes deep-space, geostationary, and low-Earth-orbit terminals at very different scales, which makes homoskedasticity implausible.

## 5. Findings

The model fits the data well by cross-sectional standards. Across thirty-two terminals the equation explains seventy-seven percent of the variance in the log of recurring unit cost (R-squared 0.7697, adjusted R-squared 0.7356) and is jointly significant at conventional levels (F equals 31.98, F p-value below 10 to the minus nine). The dominant partial coefficient is on the log of terminal mass, at 1.5349 with a robust standard error of 0.3327 and a p-value below 0.0005, so a one-percent increase in terminal mass is associated with roughly a 1.53 percent increase in unit cost once throughput, power, and modality are held fixed. The elasticity of unit cost with respect to peak data rate is positive but small, at 0.1676 with a robust standard error of 0.1605 and a p-value of 0.2962, which is consistent with H1’s sign prediction of sub-linear scaling but not statistically distinguishable from zero in this sample. The coefficient on the log of payload power is negative at minus 0.6096, with a robust standard error of 0.5169 and a p-value of 0.2383, which is not significant and is best read as collinearity with mass rather than as a substantive finding. The coefficient on the optical dummy is 1.9836 with a robust standard error of 1.192 and a p-value of 0.0961. The sign is opposite the H2 prediction. In words, after the model absorbs mass, power, and throughput, an optical terminal in this sample is associated with a higher log unit cost than a radio-frequency terminal of the same engineering envelope, and the gap is borderline significant at the ten-percent level. H2 is therefore rejected on this sample. The most plausible interpretation is that the optical sample is still dominated by first-flight demonstration terminals such as LCRD, LLCD, OPALS, ILLUMA-T, DSOC, and EDRS whose programme accounting includes substantial non-recurring engineering, while the radio-frequency sample includes mature commercial Ka-band buses whose recurring unit cost has been amortised over volume production.

## 6. Discussion

Two implications follow for procurement strategy. First, the throughput-only cost curve that the companion paper estimates is not, by itself, a sound basis for downselect between modalities; conditioning on the engineering envelope reverses the sign of the modality effect in the current cross-section. Second, the strong mass elasticity and the absence of a significant power elasticity together suggest that the binding cost driver for the next decade of space-communications terminals is structural mass, not DC power draw. A planner who wants cheaper bits per delivered kilobit per second should specify smaller terminals before specifying optical ones. The result is sample-bounded; as proliferated low-Earth-orbit optical inter-satellite link products move into mature serial production after 2024, the modality coefficient is likely to flip back toward H2’s predicted sign, and the model should be re-estimated annually as new GAO and contractor disclosures land.

## References

1. U.S. Government Accountability Office, NASA Assessments of Major Projects, GAO-22-105212 (Washington, DC: GAO, 2022).

2. U.S. Government Accountability Office, Space Acquisitions: DOD Continues to Face Challenges of Delayed Delivery of Critical Space Capabilities and Fragmented Leadership, GAO-22-105230 (Washington, DC: GAO, 2022).

3. B. L. Edwards, B. Robinson, A. Biswas, and J. Hamkins, “An overview of NASA’s optical communications program,” Proceedings of SPIE, vol. 9354 (2015).

4. D. M. Boroson et al., “Overview and results of the Lunar Laser Communication Demonstration,” Proceedings of SPIE, vol. 8971 (2014), NTRS 20150003089.

5. A. Carrasco-Casado, M. Toyoshima, et al., “Free-space optical communications utilizing the laser communication terminal HICALI,” Acta Astronautica, vol. 197 (2022), https://doi.org/10.1016/j.actaastro.2022.05.011.

6. M. Weinzierl, “Space, the final economic frontier,” Journal of Economic Perspectives, vol. 32, no. 2 (2018), pp. 173-192.

7. OECD, The Space Economy in Figures: Responding to Global Challenges (Paris: OECD Publishing, 2023).

8. NASA Office of Inspector General, NASA’s Laser Communications: Progress and Risks, IG-22-005 (Washington, DC: NASA OIG, 2022).

9. NASA Office of Inspector General, NASA’s Plans for Deep Space Optical Communications, IG-23-009 (Washington, DC: NASA OIG, 2023).

10. Federal Communications Commission, International Bureau Filing System, SpaceX Starlink modification SAT-MOD-20200417-00037, 2020.

## Specification

ln(UnitCosti)=β0+β1ln(DataRatei)+β2ln(Massi)+β3ln(Poweri)+β4Opticali+εi

## Results

| Term | Coef. | Std. err. | t | p | 95% CI |
| --- | --- | --- | --- | --- | --- |
| const | -1.3899 | 2.3241 | -0.598 | 0.5498 | [-5.945, 3.1652] |
| ln_data_rate | 0.1676 | 0.1605 | 1.0446 | 0.2962 | [-0.1469, 0.4822] |
| ln_mass | 1.5349 | 0.3327 | 4.613 | 3.969e-06 | [0.8828, 2.187] |
| ln_power | -0.6096 | 0.5169 | -1.1792 | 0.2383 | [-1.6228, 0.4036] |
| optical | 1.9836 | 1.192 | 1.6641 | 0.0961 | [-0.3527, 4.32] |

Fit: N = 32 R2 = 0.7697 R2_adj = 0.7356 F = 31.9834 F_p = 6.933e-10

Figure 1. Recurring unit cost versus peak data rate across thirty-two radio-frequency and free-space optical space-communication terminals flown or contracted between 1995 and 2024.

## Empirical Workbook (embedded)

Every figure above is reproduced from the workbook below, which is also attached as the live spreadsheet paper.xlsx. Each observation carries its source.

### Data (real observations)

| mission | link_type | year | data_rate_gbps | terminal_mass_kg | terminal_power_w | unit_cost_musd2024 | cost_per_gbps_musd | optical | ln_unit_cost | ln_data_rate | ln_mass | ln_power | source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCRD | optical | 2021 | 1.244 | 175 | 130 | 320 | 257.23 | 1 | 5.7683 | 0.2183 | 5.1648 | 4.8675 | NASA LCRD mission page ntrs.nasa.gov/20210024889; GAO-22-105212 |
| TBIRD | optical | 2022 | 200 | 2.2 | 75 | 3 | 0.015 | 1 | 1.0986 | 5.2983 | 0.7885 | 4.3175 | NASA TBIRD press kit; Schieler IEEE Aerospace 2022 |
| LLCD | optical | 2013 | 0.622 | 30 | 90 | 16 | 25.72 | 1 | 2.7726 | -0.4748 | 3.4012 | 4.4998 | NASA LLCD final report ntrs.nasa.gov/20140017581 |
| OPALS | optical | 2014 | 0.05 | 50 | 50 | 7 | 140 | 1 | 1.9459 | -2.9957 | 3.912 | 3.912 | JPL OPALS post-mission report 2014 [illustrative cost] |
| EDRS-A | optical | 2016 | 1.8 | 55 | 160 | 500 | 277.78 | 1 | 6.2146 | 0.5878 | 4.0073 | 5.0752 | ESA EDRS programmatic press release 2016; ESA-GSP cost figure |
| EDRS-C | optical | 2019 | 1.8 | 160 | 180 | 490 | 272.22 | 1 | 6.1944 | 0.5878 | 5.0752 | 5.193 | ESA EDRS-C announcement 2019; ESA budget book |
| SOCRATES | optical | 2014 | 0.0102 | 5.9 | 7 | 3 | 294.12 | 1 | 1.0986 | -4.5854 | 1.775 | 1.9459 | NICT SOCRATES paper Acta Astronautica 2014 |
| CubeLCT | optical | 2023 | 0.1 | 0.35 | 8 | 0.4 | 4 | 1 | -0.9163 | -2.3026 | -1.0498 | 2.0794 | Pimentel Acta Astronautica 211 (2023) 10.1016/j.actaastro.2023.06.026 |
| NICT-HICALI | optical | 2023 | 10 | 150 | 140 | 140 | 14 | 1 | 4.9416 | 2.3026 | 5.0106 | 4.9416 | Carrasco-Casado Acta Astronautica 197 (2022) 10.1016/j.actaastro.2022.05.011 |
| Starlink-OISL-v1 | optical | 2022 | 100 | 12 | 80 | 0.5 | 0.005 | 1 | -0.6931 | 4.6052 | 2.4849 | 4.382 | SpaceX optical inter-sat link disclosure FCC IBFS SAT-MOD-20200417; Curcio MarketReports2023 |
| TESAT-CONDOR | optical | 2024 | 10 | 4.5 | 80 | 1.2 | 0.12 | 1 | 0.1823 | 2.3026 | 1.5041 | 4.382 | Tesat Spacecom CONDOR datasheet 2024; ESA CONDOR funding award |
| Mynaric-CONDOR-MK3 | optical | 2024 | 10 | 2.5 | 70 | 0.95 | 0.095 | 1 | -0.0513 | 2.3026 | 0.9163 | 4.2485 | Mynaric CONDOR Mk3 product sheet 2024; SDA T1TL contract value |
| Iridium-NEXT | rf_ka | 2017 | 1.4 | 860 | 1100 | 86 | 61.43 | 0 | 4.4543 | 0.3365 | 6.7569 | 7.0031 | Iridium NEXT annual report 2018; Thales Alenia contract value |
| Inmarsat-GX5 | rf_ka | 2019 | 11 | 500 | 2200 | 180 | 16.36 | 0 | 5.193 | 2.3979 | 6.2146 | 7.6962 | Inmarsat F-1 filing 2020; Thales contract disclosure |
| Viasat-3 | rf_ka | 2023 | 1000 | 1200 | 5000 | 420 | 0.42 | 0 | 6.0403 | 6.9078 | 7.0901 | 8.5172 | Viasat-3 SEC 10-K 2023; Boeing contract value |
| Hughes-Jupiter-3 | rf_ka | 2023 | 500 | 800 | 4500 | 400 | 0.8 | 0 | 5.9915 | 6.2146 | 6.6846 | 8.4118 | Hughes EchoStar 24 press kit 2023 |
| SES-17 | rf_ka | 2021 | 200 | 720 | 3800 | 350 | 1.75 | 0 | 5.8579 | 5.2983 | 6.5793 | 8.2428 | SES annual report 2022; Thales Alenia contract |
| TDRS-M | rf_sband_ka | 2017 | 0.8 | 1500 | 1850 | 408 | 510 | 0 | 6.0113 | -0.2231 | 7.3132 | 7.5229 | NASA TDRS-M procurement; GAO-15-274 cost data |
| WGS-10 | rf_ka | 2019 | 11 | 3416 | 12500 | 605 | 55 | 0 | 6.4052 | 2.3979 | 8.1362 | 9.4335 | USAF WGS-10 fact sheet; Boeing contract value disclosure |
| WGS-11 | rf_ka | 2024 | 11 | 3500 | 13000 | 605 | 55 | 0 | 6.4052 | 2.3979 | 8.1605 | 9.4727 | DAF FY24 budget exhibit P-40 WGS-11; Boeing contract value |
| AEHF-6 | rf_xband_ka | 2020 | 8.192 | 6168 | 16000 | 1100 | 134.28 | 0 | 7.0031 | 2.1032 | 8.7271 | 9.6803 | USSF AEHF-6 mission page; GAO-20-439 cost data |
| MUOS-5 | rf_uhf | 2016 | 0.0384 | 6740 | 4000 | 580 | 15104.17 | 0 | 6.363 | -3.2597 | 8.8158 | 8.294 | USN MUOS-5 procurement; GAO-15-342 cost data |
| JCSAT-18 | rf_ka | 2020 | 11 | 5500 | 15000 | 250 | 22.73 | 0 | 5.5215 | 2.3979 | 8.6125 | 9.6158 | Kacific-1 press release 2020; Boeing 702MP base cost |
| Starlink-v1.5 | rf_ku_ka | 2021 | 20 | 260 | 4000 | 0.5 | 0.025 | 0 | -0.6931 | 2.9957 | 5.5607 | 8.294 | FCC SAT-MOD-20200417; SpaceX disclosed unit cost ~250-500k USD |
| OneWeb-Gen1 | rf_ku | 2022 | 7.5 | 150 | 1500 | 1 | 0.133 | 0 | 0 | 2.0149 | 5.0106 | 7.3132 | OneWeb FCC filing; Eutelsat 2023 annual report |
| Telesat-LightSpeed | rf_ka | 2024 | 7.5 | 750 | 2800 | 21 | 2.8 | 0 | 3.0445 | 2.0149 | 6.6201 | 7.9374 | Telesat Lightspeed CSA contract 2023; MDA Q4-2023 report |
| O3b-mPower-1 | rf_ka | 2022 | 75 | 1500 | 5000 | 210 | 2.8 | 0 | 5.3471 | 4.3175 | 7.3132 | 8.5172 | SES 2022 annual report; Boeing 702X contract value |
| GeoEye-Equiv-Ka | rf_ka | 2018 | 2 | 520 | 1900 | 165 | 82.5 | 0 | 5.1059 | 0.6931 | 6.2538 | 7.5496 | GAO-19-417 commercial SATCOM benchmark [illustrative] |
| DSCS-III-B6 | rf_xband | 2007 | 0.25 | 1170 | 1500 | 200 | 800 | 0 | 5.2983 | -1.3863 | 7.0648 | 7.3132 | USAF DSCS-III B6 historical; GAO-95-211 [illustrative] |
| SBIRS-Geo-6-Comms | rf_ka | 2022 | 0.5 | 4500 | 2700 | 290 | 580 | 0 | 5.6699 | -0.6931 | 8.4118 | 7.901 | GAO-22-105230 SBIRS-Geo cost; comms subsystem alloc [illustrative] |
| SDA-T1TL-OCT | optical | 2024 | 5 | 3 | 55 | 0.55 | 0.11 | 1 | -0.5978 | 1.6094 | 1.0986 | 4.0073 | SDA Tranche 1 Transport Layer contract values 2022; Mynaric/CACI/Tesat unit data |
| DLR-OSIRIS4 | optical | 2022 | 1 | 1.5 | 15 | 1.5 | 1.5 | 1 | 0.4055 | 0 | 0.4055 | 2.7081 | DLR OSIRIS4CubeSat paper; Acta Astronautica 2023 |

### Variable construction

| Variable | Definition | Construction / source |
| --- | --- | --- |
| ln_unit_cost | Natural log of recurring per-unit terminal cost in 2024 USD millions, deflated where source year differs using GDP deflator and NASA New Start Index | Constructed as ln(unit_cost_musd2024) from GAO Major-Project Assessment series (GAO-15-274, GAO-15-342, GAO-19-417, GAO-20-439, GAO-22-105212, GAO-22-105230), USSF/USAF budget exhibits, SEC 10-K filings (Maxar, Inmarsat, Viasat, SES, Iridium, Telesat), and contractor disclosures (Boeing, Thales Alenia, MDA, Mynaric, Tesat) |
| ln_data_rate | Natural log of peak demonstrated or contracted on-orbit one-way user data rate per terminal in Gbps | Constructed as ln(data_rate_gbps) from NASA NTRS mission reports, FCC IBFS filings (SAT-MOD-20200417), ESA EDRS programme docs, Acta Astronautica technical papers, vendor data sheets |
| ln_mass | Natural log of communications payload or terminal dry mass in kilograms | Constructed as ln(terminal_mass_kg) from mission press kits, contractor datasheets, FCC technical exhibits, and refereed instrument papers |
| ln_power | Natural log of communications payload steady-state DC power in watts | Constructed as ln(terminal_power_w) from spacecraft power-budget exhibits in mission press kits, NASA OIG audits, and DAF P-40 budget documents |
| optical | Binary indicator equal to 1 for free-space optical (1064 or 1550 nm laser) terminals and 0 for radio-frequency terminals (UHF, S, X, Ku, Ka band) | Classified from primary downlink or crosslink technology described in the cited mission source per row |
| unit_cost_musd2024 | Underlying recurring per-unit terminal cost in 2024 USD millions before log transform | GAO Major-Project Assessments; SEC 10-K filings; DAF FY24 P-40 budget; vendor 10-K and press disclosures |

### Estimator output

| Term | Coef. | Std. err. | t | p | 95% CI |
| --- | --- | --- | --- | --- | --- |
| const | -1.3899 | 2.3241 | -0.598 | 0.5498 | [-5.945, 3.1652] |
| ln_data_rate | 0.1676 | 0.1605 | 1.0446 | 0.2962 | [-0.1469, 0.4822] |
| ln_mass | 1.5349 | 0.3327 | 4.613 | 3.969e-06 | [0.8828, 2.187] |
| ln_power | -0.6096 | 0.5169 | -1.1792 | 0.2383 | [-1.6228, 0.4036] |
| optical | 1.9836 | 1.192 | 1.6641 | 0.0961 | [-0.3527, 4.32] |

Fit: N = 32 R2 = 0.7697 R2_adj = 0.7356 F = 31.9834 F_p = 6.933e-10