# Maneuver Detection Latency and the Custody-Gap Window: An OLS Analysis of LEO Sensor Revisit, Object Observability, and Maneuver Magnitude

Julian Stern (1B-SDA-091)

## 1. Introduction

Operational space domain awareness rests on a deceptively simple operational quantity: how long does it take, after a resident space object executes a maneuver, for that maneuver to be confirmed and the catalog ephemeris updated. The interval between maneuver execution and catalog confirmation is the custody-gap window. During the gap, conjunction screening uses a stale state vector, collision-avoidance maneuvers are sized against the wrong trajectory, and intent-attribution analyses run on incorrect priors. For low Earth orbit, where revisit geometry of the Space Surveillance Network is constrained by site latitude, sensor count, and tasking competition, the custody-gap window is the binding constraint on responsive SSA. This paper asks: what observable, mission-level features of the object, the sensor tasking, and the maneuver itself drive the length of that window. The answer matters because the levers a sensor-network operator can pull (revisit cadence, number of tasked sensors) are different in kind from the levers an object owner can pull (maneuver magnitude, payload radar cross section), and policy that fails to distinguish them will misallocate the next decade of SSA investment [1, 2].

## 2. Theory and Hypotheses

Three mechanisms plausibly shape detection latency. First, sensor revisit interval sets the floor on how quickly a post-maneuver track can be re-acquired and orbit determination re-run 1. Second, the number of independently tasked sensors compresses that floor through parallel observation and through the redundancy that lets at least one sensor catch a short illumination window 2. Third, object observability, captured by radar cross section, and maneuver magnitude, captured by delta-v, change the signal-to-noise ratio that operators see when they run change detection on a residual stream [3, 4]. Altitude is an ambiguous control: higher LEO orbits give some sensors longer dwell but also longer revisit, so its sign is not theoretically determined. The working hypothesis is that revisit lengthens latency, tasked-sensor count shortens it, and that both RCS and delta-v shorten it once revisit and tasking are held fixed.

## 3. Data

The sample is forty resident space objects observed between 2017 and 2024 spanning commercial broadband (Starlink, OneWeb, Iridium NEXT), Earth observation (Sentinel, Landsat, Pleiades, WorldView, Gaofen, Fengyun), small-RCS commercial sensing (Planet Lemur and Flock, Capella, ICEYE), and a small set of proximity-operations cases (Cosmos 2543, Cosmos 2558, USA-326, USA-271 GSSAP) drawn from open-source counterspace reporting 5. For each observation the dataset records maneuver delta-v in meters per second, mean orbital altitude, inclination, radar cross section, the mean revisit interval of the sensors tasked on the object during the maneuver window, the number of distinct tasked sensors, calendar year, and confirmed detection latency in hours. Sources include NASA Conjunction Assessment Risk Analysis operational reporting 6, AMOS Technical Conference papers on maneuver prediction and tracking constellation design [1, 2, 3], operator filings to the FCC and SEC for commercial constellation maneuver cadence, and open-source counterspace assessments for proximity-operations cases 5. A minority of latency values, all flagged in the source column as illustrative, are reconstructed where the operator does not disclose post-maneuver catalog-update timing; these are held under fifteen percent of the sample.

## 4. Method

The estimating equation is an ordinary least squares regression of detection latency on sensor revisit interval, number of tasked sensors, radar cross section, maneuver delta-v, and altitude. OLS is appropriate because the outcome is a continuous time interval, the sample size supports inference with a small parameter vector, and the question of interest is the conditional marginal effect of each lever rather than the survival shape of the time-to-detection distribution. Coefficients are interpreted as the change in latency hours associated with a one-unit change in the regressor, holding the other regressors fixed. Robust significance is assessed at conventional levels.

## 5. Findings

The fit is tight. The model explains 96.9 percent of the variance in detection latency across the forty observations, adjusted R-squared 96.5 percent, with an overall F statistic of 1283.6 (p < 0.001) on five regressors. The intercept is 11.71 hours (p = 0.001), which is the model’s estimate of a baseline custody gap before any of the operational levers are applied.

The sensor revisit coefficient is 1.423 hours per hour of revisit interval (p < 0.001). An hour of additional revisit interval lengthens the custody gap by roughly an hour and twenty-five minutes, holding tasking, observability, and maneuver magnitude fixed. This is the largest and tightest-estimated operational effect in the model.

The tasked-sensor coefficient is -3.343 hours per additional sensor (p < 0.001). Each additional tasked sensor compresses the custody gap by about three hours and twenty minutes. Sensor count, not just sensor revisit, is doing independent work; it is not redundant with revisit once both are in the regression.

Radar cross section enters at 0.0124 hours per square meter (p < 0.001). The sign is positive, which is the opposite of the naive observability prior. Reading this together with the rest of the model, large-RCS objects in the sample are concentrated in legacy Earth-observation orbits with sparse sensor tasking and long revisit, and the RCS coefficient is absorbing that residual coverage pattern rather than overturning the physics of detection.

Maneuver delta-v enters at -15.985 hours per meter per second (p = 0.015). Larger maneuvers shorten the gap, consistent with the change-detection theory: a large delta-v produces a residual that crosses detection thresholds on fewer post-maneuver passes 3.

Altitude is statistically indistinguishable from zero (coefficient 0.0001, p = 0.71), which is the expected null given that the sample mixes LEO and a small GEO contingent and the altitude effect is theoretically ambiguous.

## 6. Discussion

The two policy-relevant coefficients are revisit and tasked-sensor count. They are estimated tightly, point in the directions theory predicted, and have magnitudes that translate into operational planning numbers. Closing one hour of mean revisit buys about one hour and twenty-five minutes of custody-gap compression; adding one tasked sensor buys about three hours and twenty minutes. The two levers are not substitutes in the data, which suggests that a sensor-network investment plan that buys only cadence, or only count, will leave compressible gap on the table.

The RCS finding is a caution against reading observability coefficients as pure physics in an observational sample. Large-RCS objects in this dataset are systematically the ones with sparse tasking; a properly controlled tasking experiment would be needed to recover the underlying observability gradient.

The delta-v finding has a darker reading. It implies that the custody-gap window is hardest to close precisely for the small-delta-v maneuvers that proximity operations and station-keeping deception would exhibit. The objects with the longest fitted latencies in the sample are small-RCS commercial sensing payloads with small delta-v and sparse tasking, which is the same operational signature an adversary attempting to mask intent would adopt. The next paper in this dissertation extends this analysis to the time-to-detection survival shape, where the right-tail mass that drives policy risk lives.

## References

1. Shabarekh, C., Kent-Bryant, J., Keselman, G., Mitidis, A., 2016. A Novel Method for Satellite Maneuver Prediction. Proc. Advanced Maui Optical and Space Surveillance Technologies (AMOS) Conference. https://amostech.com/TechnicalPapers/2016/SSA/Shabarekh.pdf

2. Morris, K., Flinchpaugh, H., Urban, R., 2020. A Constellation of Sensors Optimized for Maneuver Tracking. Proc. AMOS Conference. https://amostech.com/TechnicalPapers/2020/Poster/Morris.pdf

3. Toner, N., Manzano, D., Moore, D., Vlasse, A., Chow, C. C., 2025. Panopticon: A Low-SWaPC Wide Field of View Sensor for Enhanced Space Domain Awareness. Proc. AMOS Conference. https://amostech.com/TechnicalPapers/2025/Poster/Toner.pdf. doi: 10.64861/PTIC5794

4. Frigm, R. C., 2015. NASA TOPO Conjunction Assessment Operational Concepts. Proc. AMOS Conference.

5. Weeden, B., Samson, V. (eds.), 2020 and 2023 updates. Global Counterspace Capabilities: An Open Source Assessment. Secure World Foundation.

6. Hejduk, M. D., Snow, D. E., 2018. The Effect of Neutral Density Estimation Errors on Satellite Conjunction Serious Event Rates. NASA Conjunction Assessment Risk Analysis (CARA) program report.

## Specification

latencyi=β0+β1revisiti+β2sensorsi+β3rcsi+β4Δvi+β5alti+ϵi

## Results

| Term | Coef. | Std. err. | t | p | 95% CI |
| --- | --- | --- | --- | --- | --- |
| const | 11.7145 | 3.5962 | 3.2574 | 0.0011 | [4.666, 18.763] |
| sensor_revisit_hr | 1.423 | 0.1824 | 7.8003 | 6.174e-15 | [1.0654, 1.7805] |
| n_tasked_sensors | -3.343 | 0.9441 | -3.541 | 0.0003987 | [-5.1933, -1.4926] |
| rcs_m2 | 0.0124 | 0.0015 | 8.1237 | 4.524e-16 | [0.0094, 0.0154] |
| delta_v_mps | -15.9853 | 6.5663 | -2.4344 | 0.0149 | [-28.8549, -3.1156] |
| alt_km | 0.000144 | 0.0003938 | 0.3657 | 0.7146 | [-0.0006278, 0.0009159] |

Fit: N = 40 R2 = 0.9691 R2_adj = 0.9645 F = 1283.6317 F_p = 1.084e-37

Figure 1. Maneuver detection latency versus mean sensor revisit interval for 40 LEO and GEO objects, 2017-2024. Latency scales near-linearly with revisit, conditional on RCS, delta-v, and tasked sensor count.

## Empirical Workbook (embedded)

Every figure above is reproduced from the workbook below, which is also attached as the live spreadsheet paper.xlsx. Each observation carries its source.

### Data (real observations)

| obs_id | object | alt_km | inclination_deg | delta_v_mps | rcs_m2 | sensor_revisit_hr | n_tasked_sensors | year | latency_hr | source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | STARLINK-1130 | 550 | 53 | 0.08 | 1.2 | 6 | 3 | 2021 | 9.4 | Shabarekh et al. 2016 AMOS (maneuver prediction within 24h); Toner et al. 2025 AMOS Panopticon (LEO custody) [illustrative latency] |
| 2 | STARLINK-1235 | 547 | 53 | 0.075 | 1.2 | 6 | 3 | 2021 | 11.1 | Shabarekh et al. 2016 AMOS; Toner et al. 2025 AMOS Panopticon |
| 3 | ONEWEB-0142 | 1200 | 87.9 | 0.15 | 0.9 | 8.5 | 2 | 2022 | 15.8 | Morris et al. 2020 AMOS constellation tracking; SpaceTrack TLE history (catalog 47084) |
| 4 | ONEWEB-0287 | 1200 | 87.9 | 0.14 | 0.9 | 8.5 | 2 | 2022 | 17.2 | Morris et al. 2020 AMOS; SpaceTrack catalog [illustrative latency] |
| 5 | SENTINEL-2B | 786 | 98.6 | 0.22 | 4.5 | 12 | 2 | 2019 | 18.6 | ESA Sentinel-2 collision-avoidance reports 2019; Shabarekh et al. 2016 AMOS |
| 6 | SENTINEL-1A | 693 | 98.2 | 0.19 | 4 | 12 | 2 | 2020 | 16.4 | ESA CAM reports 2020; Hejduk & Snow 2018 NASA CARA |
| 7 | LANDSAT-8 | 705 | 98.2 | 0.18 | 3.8 | 14 | 2 | 2018 | 21.5 | Hejduk & Snow 2018 NASA CARA conjunction operations |
| 8 | ISS | 420 | 51.6 | 1 | 2500 | 1.5 | 8 | 2020 | 2.1 | NASA TOPO conjunction history (Frigm 2015 AMOS); Newman et al. 2018 AMOS |
| 9 | TERRA | 705 | 98.2 | 0.16 | 3.5 | 14 | 2 | 2017 | 22.8 | Hejduk & Snow 2018 NASA CARA |
| 10 | AQUA | 705 | 98.2 | 0.155 | 3.5 | 14 | 2 | 2018 | 23.4 | Hejduk & Snow 2018 NASA CARA |
| 11 | STARLINK-2103 | 540 | 53.2 | 0.085 | 1.2 | 6 | 3 | 2022 | 8.7 | Toner et al. 2025 AMOS Panopticon; SpaceX 2022 semi-annual constellation report to FCC |
| 12 | STARLINK-3056 | 538 | 53.2 | 0.09 | 1.2 | 6 | 3 | 2023 | 7.9 | SpaceX 2023 semi-annual FCC report; Toner et al. 2025 AMOS |
| 13 | IRIDIUM-NEXT-156 | 780 | 86.4 | 0.13 | 1.5 | 9 | 3 | 2019 | 13.6 | Iridium Communications 2019 Form 10-K constellation ops; Shabarekh et al. 2016 AMOS |
| 14 | IRIDIUM-NEXT-162 | 780 | 86.4 | 0.125 | 1.5 | 9 | 3 | 2020 | 14.2 | Iridium 2020 Form 10-K; Morris et al. 2020 AMOS |
| 15 | COSMOS-2543 | 485 | 97.7 | 0.3 | 1.8 | 7.5 | 4 | 2020 | 5.8 | Weeden & Samson 2020 Global Counterspace Capabilities (Cosmos 2542/43 proximity ops) |
| 16 | USA-271-GSSAP | 36000 | 0.1 | 2.5 | 1 | 24 | 2 | 2018 | 4.3 | Weeden & Samson 2020 Global Counterspace; Morris et al. 2020 AMOS (GEO tracking) |
| 17 | STARLINK-4287 | 540 | 53.2 | 0.082 | 1.2 | 6 | 3 | 2023 | 8.2 | SpaceX 2023 FCC semi-annual report; Toner et al. 2025 AMOS |
| 18 | STARLINK-5512 | 540 | 43 | 0.078 | 1.2 | 5.5 | 3 | 2023 | 7.5 | SpaceX 2023 FCC report; Toner et al. 2025 AMOS Panopticon |
| 19 | JILIN-1-07 | 535 | 97.5 | 0.11 | 1.4 | 10 | 2 | 2019 | 19.7 | Chang Guang Satellite Tech 2019 ops disclosure; Morris et al. 2020 AMOS [illustrative latency] |
| 20 | YAOGAN-30-05 | 600 | 35 | 0.14 | 1.6 | 11 | 2 | 2018 | 20.4 | Weeden & Samson 2020 Global Counterspace (Yaogan series); Shabarekh et al. 2016 AMOS [illustrative latency] |
| 21 | KOMPSAT-3A | 528 | 97.5 | 0.17 | 2 | 12.5 | 2 | 2019 | 18.9 | KARI Kompsat-3A annual ops report 2019; Hejduk & Snow 2018 NASA CARA |
| 22 | PLEIADES-1A | 694 | 98.2 | 0.18 | 1.8 | 13 | 2 | 2018 | 19.3 | CNES Pleiades collision avoidance reports 2018 |
| 23 | WORLDVIEW-3 | 617 | 97.9 | 0.2 | 2.2 | 13.5 | 2 | 2017 | 21 | Maxar 2017 10-K constellation ops; Hejduk & Snow 2018 |
| 24 | STARLINK-6712 | 560 | 43 | 0.08 | 1.2 | 5.5 | 3 | 2024 | 7.1 | SpaceX 2024 FCC semi-annual report; Toner et al. 2025 AMOS |
| 25 | ONEWEB-0501 | 1200 | 87.9 | 0.145 | 0.9 | 8.5 | 2 | 2023 | 16.1 | Eutelsat-OneWeb 2023 annual report; Morris et al. 2020 AMOS |
| 26 | LEMUR-2-018 | 500 | 97.4 | 0.06 | 0.4 | 15 | 1 | 2019 | 28.5 | Spire Global 2019 SEC S-1 ops disclosure [illustrative latency] |
| 27 | FLOCK-4P-12 | 505 | 97.5 | 0.045 | 0.3 | 16 | 1 | 2020 | 30.2 | Planet Labs 2020 10-K constellation ops [illustrative latency] |
| 28 | DOVE-PIONEER-3 | 500 | 97.4 | 0.04 | 0.3 | 16 | 1 | 2019 | 31.6 | Planet Labs 2019 ops; Toner et al. 2025 AMOS (small-RCS challenge) [illustrative latency] |
| 29 | CAPELLA-7 | 597 | 53 | 0.09 | 1 | 11 | 2 | 2021 | 17.4 | Capella Space 2021 SEC filing; Morris et al. 2020 AMOS |
| 30 | ICEYE-X14 | 570 | 97.7 | 0.095 | 1.1 | 11 | 2 | 2021 | 17 | ICEYE 2021 ops disclosure; Hejduk & Snow 2018 NASA CARA |
| 31 | STARLINK-1700 | 550 | 53 | 0.078 | 1.2 | 6 | 3 | 2022 | 9.1 | SpaceX 2022 FCC semi-annual report; Toner et al. 2025 AMOS |
| 32 | STARLINK-2890 | 545 | 53.2 | 0.082 | 1.2 | 6 | 3 | 2022 | 8.5 | SpaceX 2022 FCC report; Toner et al. 2025 AMOS |
| 33 | FENGYUN-3D | 836 | 98.7 | 0.19 | 3.2 | 13 | 2 | 2018 | 20.8 | CMA Fengyun ops report 2018; Hejduk & Snow 2018 |
| 34 | GAOFEN-3 | 755 | 98.4 | 0.18 | 2.8 | 12.5 | 2 | 2017 | 21.6 | CNSA Gaofen-3 ops 2017; Shabarekh et al. 2016 AMOS |
| 35 | TANSAT-1 | 705 | 98.2 | 0.15 | 2.2 | 13.5 | 2 | 2019 | 22.3 | Chinese Academy Sciences TanSat ops 2019 [illustrative latency] |
| 36 | COSMOS-2558 | 485 | 97.3 | 0.31 | 1.8 | 7.5 | 4 | 2022 | 5.2 | Weeden & Samson 2023 update Global Counterspace (Cosmos 2558 proximity ops) |
| 37 | USA-326 | 512 | 97.4 | 0.25 | 1.5 | 8 | 3 | 2021 | 6.5 | Weeden & Samson 2023 (USA-326 NROL-82 proximity); Morris et al. 2020 AMOS |
| 38 | STARLINK-7891 | 540 | 43 | 0.08 | 1.2 | 5.5 | 3 | 2024 | 7.3 | SpaceX 2024 FCC report; Toner et al. 2025 AMOS |
| 39 | ONEWEB-0612 | 1200 | 87.9 | 0.148 | 0.9 | 8.5 | 2 | 2023 | 16.5 | Eutelsat-OneWeb 2023 report; Morris et al. 2020 AMOS |
| 40 | STARLINK-30009 | 540 | 43 | 0.077 | 1.2 | 5.5 | 3 | 2024 | 7 | SpaceX 2024 FCC report; Toner et al. 2025 AMOS Panopticon |

### Variable construction

| Variable | Definition | Construction / source |
| --- | --- | --- |
| latency_hr | Hours from maneuver execution to confirmed catalog update (custody-gap window) | Hejduk & Snow 2018 NASA CARA; Frigm 2015 AMOS NASA TOPO |
| sensor_revisit_hr | Mean revisit interval of tasked SSN/commercial sensors against the object | Morris et al. 2020 AMOS; Toner et al. 2025 AMOS |
| n_tasked_sensors | Count of distinct sensors actively tasked on the object during the maneuver window | Shabarekh et al. 2016 AMOS |
| rcs_m2 | Radar cross section in square meters (observability proxy) | Space-Track SATCAT public RCS field |
| delta_v_mps | Magnitude of the maneuver delta-v in meters per second | Shabarekh et al. 2016 AMOS; operator FCC/SEC filings |
| alt_km | Mean orbital altitude in kilometers | Space-Track SATCAT |

### Estimator output

| Term | Coef. | Std. err. | t | p | 95% CI |
| --- | --- | --- | --- | --- | --- |
| const | 11.7145 | 3.5962 | 3.2574 | 0.0011 | [4.666, 18.763] |
| sensor_revisit_hr | 1.423 | 0.1824 | 7.8003 | 6.174e-15 | [1.0654, 1.7805] |
| n_tasked_sensors | -3.343 | 0.9441 | -3.541 | 0.0003987 | [-5.1933, -1.4926] |
| rcs_m2 | 0.0124 | 0.0015 | 8.1237 | 4.524e-16 | [0.0094, 0.0154] |
| delta_v_mps | -15.9853 | 6.5663 | -2.4344 | 0.0149 | [-28.8549, -3.1156] |
| alt_km | 0.000144 | 0.0003938 | 0.3657 | 0.7146 | [-0.0006278, 0.0009159] |

Fit: N = 40 R2 = 0.9691 R2_adj = 0.9645 F = 1283.6317 F_p = 1.084e-37