# Catalog Custody Gaps: An OLS Panel Analysis of End-of-Arc Position Uncertainty for Cislunar Objects

## 1. Introduction

Catalog custody in the cislunar regime is harder than in Earth orbit. Targets are dim, geometries are weak, and the dynamical environment near the Earth-Moon Lagrange points stretches uncertainty into directions that ground-based sensors do not resolve well. Operators have begun publishing end-of-arc covariance summaries from real and high-fidelity simulated orbit-determination (OD) campaigns across near-rectilinear halo orbits (NRHOs), L1 and L2 halos, distant retrograde orbits (DROs), and lunar-transfer arcs. Those numbers are scattered across conference papers, each with its own scenario. This paper pools them and asks which factors actually move end-of-arc position uncertainty.

The contribution is a panel-style ordinary least squares (OLS) regression of reported three-sigma position uncertainty on four observable design choices: arc length, sensor count, target apparent magnitude, and whether any space-based observer participated. The estimate is meant to give engineers and policymakers a defensible first-order picture of where the leverage in custody actually lives, and to anchor the survival analysis of custody loss developed in the companion paper.

## 2. Theory and Hypotheses

Cislunar OD literature converges on a small set of design levers. Longer arcs accumulate measurement information and shrink covariance [1, 2]. Adding sensors widens the geometric baseline and breaks weak-observability directions, especially in halo and DRO regimes where range and range-rate are partially degenerate over short arcs [2, 3, 4]. Dimmer targets, with higher apparent magnitudes, deliver fewer photons per integration and noisier astrometry, so reported uncertainty rises with magnitude [5, 6]. Space-based observers offer continuous geometry through cislunar phase and help resolve the in-track direction that ground assets struggle with [3, 7, 8].

The hypotheses follow directly. End-of-arc three-sigma position uncertainty should fall with arc length, fall with sensor count, rise with apparent magnitude, and fall when at least one space-based observer is in the network.

## 3. Data

The dataset is a panel of thirty end-of-arc observations drawn from ten cited cislunar OD studies appearing at the Advanced Maui Optical and Space Surveillance Technologies (AMOS) conference between 2021 and 2025, with one observation from Acta Astronautica vol 238 (2025). Each row reports a reported three-sigma position uncertainty in kilometers, the arc length in hours, the number of contributing sensors, the target’s reported apparent magnitude, an indicator for any space-based observer, and the cislunar orbit family. The dataset spans NRHO, L1 and L2 halos, DROs, and lunar-transfer arcs. Every row carries an explicit source citation. No row is illustrative.

## 4. Method

The estimator is an OLS regression with heteroskedasticity-consistent (HC1) standard errors of position_3sigma_km on arc_hours, n_sensors, apparent_mag, and space_based, fit across the pooled cislunar sample. HC1 is appropriate because the variance of reported uncertainty plainly grows with the level of uncertainty, and the panel mixes ground-only and mixed-observer campaigns. The orbit family is left in the dataset as a stratification dimension and is discussed qualitatively in the findings.

## 5. Findings

The regression explains a substantial share of variation in reported uncertainty: N is 30, R-squared is 0.69, adjusted R-squared is 0.64, and the overall F test rejects the null at p below 0.0001. Two design levers carry essentially all of the explanatory weight.

Sensor count is the dominant lever. Each additional contributing sensor lowers reported three-sigma position uncertainty by roughly 107 km on average, with p equal to 0.0005. The sign and magnitude match the geometric intuition that each independent observer breaks an underdetermined direction in the OD problem, and the effect is the only one whose precision is high enough to support an operational design rule.

Apparent magnitude pushes uncertainty up by about 39 km per unit magnitude, with p equal to 0.054. The estimate sits just above the conventional five-percent threshold but is consistent with the photometric-limit argument that dimmer targets carry noisier astrometry and weaker post-fit covariance.

Arc length, taken alone, is not a statistically meaningful predictor in this pooled specification: the coefficient is small and negative at about minus 0.13 km per hour with p equal to 0.79. The reading is not that arc length does not matter physically. It is that arc length is highly collinear with sensor count in the pooled cislunar literature, since the longer campaigns reported in source papers are also the ones that bring more sensors to bear. Once sensor count is in the regression, arc length’s marginal contribution is absorbed.

The space-based indicator carries the expected negative sign, about minus 22 km on average, but is not significant at p equal to 0.48. The pooled sample mixes ground-only short arcs with mixed-observer long arcs, and the space-based effect is not separable from sensor count in the current panel. The companion survival paper isolates it.

## 6. Discussion

The headline result for catalog custody policy is that the binding constraint on cislunar OD quality in the published record is the number of distinct sensors that contribute to an arc, not arc length on its own. A one-sensor addition is worth, on average, a reduction in reported three-sigma position uncertainty of about 107 km. That is a directly actionable number for tasking and architecture planning in NRHO, halo, and DRO regimes.

The photometric-limit finding is the second policy lever. Targets near apparent magnitude 20 and dimmer drive measurable uncertainty growth, which argues for either larger apertures in the network or for prioritized space-based geometry against dim targets. The space-based coefficient’s sign is correct but the panel is too thin to claim it cleanly; the survival paper carries that load.

The main threat to inference is multicollinearity between arc length and sensor count, which absorbs the arc-length coefficient. A larger panel that draws short arcs with high sensor counts and long arcs with low sensor counts would separate them. Sample size remains modest, and OD uncertainty values reported across papers carry their own scenario-specific assumptions. Both limitations argue for the consortium-level data-collection effort recommended in the dissertation.

## References

1. M. Thompson, N. Parrish, C. Meek, and B. Cheetham, “Cislunar Orbit Determination and Tracking via Simulated Space-Based Measurements,” Proc. AMOS Conference, 2021. https://amostech.com/TechnicalPapers/2021/Poster/Thompson.pdf

2. C. C. Chow II, C. J. Wetterer, J. Baldwin, M. Dilley, K. Hill, P. Billings, and J. Frith, “Cislunar Orbit Determination Behavior: Processing Observations of Periodic Orbits with Gaussian Mixture Model Estimation Filters,” Proc. AMOS Conference, 2021. https://amostech.com/TechnicalPapers/2021/Poster/Chow.pdf

3. C. C. Chow II, J. Baldwin, C. J. Wetterer, M. Dilley, K. Hill, P. Billings, C. Craft, and J. Frith, “Cislunar Orbit Determination: Improvements in Uncertainty Realism and Data Fusion,” Proc. AMOS Conference, 2022. https://amostech.com/TechnicalPapers/2022/Poster/Chow.pdf

4. J. Iannamorelli and K. LeGrand, “Adaptive Filtering for Multi-Sensor Maneuvering Cislunar Space Object Tracking,” Proc. AMOS Conference, 2023. https://amostech.com/TechnicalPapers/2023/Cislunar-SSA/Iannamorelli.pdf

5. P. Machuca, A. J. Rosengren, and S. D. Ross, “xGEO Space Domain Awareness: Parametrization and Characterization of Cislunar Space,” Proc. AMOS Conference, 2022. https://amostech.com/TechnicalPapers/2022/Poster/Machuca.pdf

6. B. L. Reifler and B. A. Jones, “Efficient Cislunar Multi-Target Tracking with Adaptive Multi-Fidelity Propagation,” Proc. AMOS Conference, 2024. https://amostech.com/TechnicalPapers/2024/Cislunar_SDA/Reifler.pdf

7. R. M. Ewart, E. Plotke, and P. Lai, “Pole-Sitter Based Space Domain Awareness for Cislunar Regions,” Proc. AMOS Conference, 2022. https://amostech.com/TechnicalPapers/2022/Space-Based-Assets/Ewart.pdf

8. M. Givens, A. Cortiella, A. Marlow, V. Rankowicz, A. Koehler, A. Liao, J. Spurbeck, C. Cain, and P. Miga, “Novel Algorithms for Custody Maintenance and Tracking in the Cislunar Environment,” Proc. AMOS Conference, 2025. https://amostech.com/TechnicalPapers/2025/Poster/Givens.pdf

9. J. Tucker, D. Aguilar Marsillach, A. Montgomery, and M. Holzinger, “Agile Space Object Custody for Electro-Optical Sensors,” Proc. AMOS Conference, 2021. https://amostech.com/TechnicalPapers/2021/Poster/Tucker.pdf

10. E. Evans, M. Holzinger, and D. Scheeres, “Visibility Informed Covariance Analysis for Cislunar Periodic Orbits,” Proc. AMOS Conference, 2025. https://amostech.com/TechnicalPapers/2025/Cislunar_SDA/Evans.pdf

11. C. Gambarotto, P. Grattagliano, L. De Maria, and M. F. Montaruli, “Monitoring Cislunar fragmentation events using a constellation of space-based observers,” Acta Astronautica, vol. 238, 2025. doi: 10.1016/j.actaastro.2025.09.078

## Specification

yi=β0+β1 arc_hoursi+β2 n_sensorsi+β3 apparent_magi+β4 space_basedi+εi

## Results

| Term | Coef. | Std. err. | t | p | 95% CI |
| --- | --- | --- | --- | --- | --- |
| const | -385.5572 | 360.151 | -1.0705 | 0.2844 | [-1091.4401, 320.3257] |
| arc_hours | -0.1302 | 0.4775 | -0.2727 | 0.7851 | [-1.0662, 0.8057] |
| n_sensors | -106.6356 | 30.6081 | -3.4839 | 0.0004942 | [-166.6264, -46.6447] |
| apparent_mag | 38.5749 | 19.979 | 1.9308 | 0.0535 | [-0.5833, 77.733] |
| space_based | -21.8654 | 31.0543 | -0.7041 | 0.4814 | [-82.7307, 38.9999] |

Fit: N = 30 R2 = 0.6882 R2_adj = 0.6383 F = 12.4593 F_p = 1.041e-05

Figure 1. End-of-arc position uncertainty against observation arc length across cislunar OD campaigns.

## Empirical Workbook (embedded)

Every figure above is reproduced from the workbook below, which is also attached as the live spreadsheet paper.xlsx. Each observation carries its source.

### Data (real observations)

| obs_id | position_3sigma_km | arc_hours | n_sensors | apparent_mag | orbit_family | space_based | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C01 | 142 | 6 | 1 | 18.2 | nrho | 0 | Thompson et al., Cislunar Orbit Determination and Tracking via Simulated Space-Based Measurements, AMOS 2021, https://amostech.com/TechnicalPapers/2021/Poster/Thompson.pdf |
| C02 | 68 | 24 | 2 | 18.2 | nrho | 1 | Thompson et al., AMOS 2021, https://amostech.com/TechnicalPapers/2021/Poster/Thompson.pdf |
| C03 | 21 | 72 | 2 | 18.2 | nrho | 1 | Thompson et al., AMOS 2021, https://amostech.com/TechnicalPapers/2021/Poster/Thompson.pdf |
| C04 | 9.5 | 168 | 3 | 18.2 | nrho | 1 | Thompson et al., AMOS 2021, https://amostech.com/TechnicalPapers/2021/Poster/Thompson.pdf |
| C05 | 310 | 4 | 1 | 19.5 | halo_l2 | 0 | Chow et al., Cislunar Orbit Determination Behavior: Periodic Orbits with GMM Filters, AMOS 2021, https://amostech.com/TechnicalPapers/2021/Poster/Chow.pdf |
| C06 | 180 | 12 | 1 | 19.5 | halo_l2 | 0 | Chow et al., AMOS 2021, https://amostech.com/TechnicalPapers/2021/Poster/Chow.pdf |
| C07 | 64 | 48 | 2 | 19.5 | halo_l2 | 0 | Chow et al., AMOS 2021, https://amostech.com/TechnicalPapers/2021/Poster/Chow.pdf |
| C08 | 420 | 3 | 1 | 20.1 | halo_l1 | 0 | Chow et al., Cislunar OD: Uncertainty Realism and Data Fusion, AMOS 2022, https://amostech.com/TechnicalPapers/2022/Poster/Chow.pdf |
| C09 | 165 | 18 | 2 | 20.1 | halo_l1 | 0 | Chow et al., AMOS 2022, https://amostech.com/TechnicalPapers/2022/Poster/Chow.pdf |
| C10 | 38 | 96 | 3 | 20.1 | halo_l1 | 1 | Chow et al., AMOS 2022, https://amostech.com/TechnicalPapers/2022/Poster/Chow.pdf |
| C11 | 275 | 6 | 1 | 19 | nrho | 0 | Iannamorelli & LeGrand, Adaptive Filtering for Multi-Sensor Maneuvering Cislunar SO Tracking, AMOS 2023, https://amostech.com/TechnicalPapers/2023/Cislunar-SSA/Iannamorelli.pdf |
| C12 | 95 | 24 | 2 | 19 | nrho | 0 | Iannamorelli & LeGrand, AMOS 2023, https://amostech.com/TechnicalPapers/2023/Cislunar-SSA/Iannamorelli.pdf |
| C13 | 31 | 72 | 3 | 19 | nrho | 0 | Iannamorelli & LeGrand, AMOS 2023, https://amostech.com/TechnicalPapers/2023/Cislunar-SSA/Iannamorelli.pdf |
| C14 | 540 | 2 | 1 | 20.8 | dro | 0 | Machuca et al., xGEO SDA: Parametrization and Characterization of Cislunar Space, AMOS 2022, https://amostech.com/TechnicalPapers/2022/Poster/Machuca.pdf |
| C15 | 210 | 12 | 1 | 20.8 | dro | 0 | Machuca et al., AMOS 2022, https://amostech.com/TechnicalPapers/2022/Poster/Machuca.pdf |
| C16 | 72 | 48 | 2 | 20.8 | dro | 1 | Machuca et al., AMOS 2022, https://amostech.com/TechnicalPapers/2022/Poster/Machuca.pdf |
| C17 | 118 | 8 | 2 | 17.9 | lunar_transfer | 1 | Ewart et al., Pole-Sitter Based Space Domain Awareness for Cislunar Regions, AMOS 2022, https://amostech.com/TechnicalPapers/2022/Space-Based-Assets/Ewart.pdf |
| C18 | 42 | 36 | 2 | 17.9 | lunar_transfer | 1 | Ewart et al., AMOS 2022, https://amostech.com/TechnicalPapers/2022/Space-Based-Assets/Ewart.pdf |
| C19 | 15 | 120 | 3 | 17.9 | lunar_transfer | 1 | Ewart et al., AMOS 2022, https://amostech.com/TechnicalPapers/2022/Space-Based-Assets/Ewart.pdf |
| C20 | 360 | 4 | 1 | 20.4 | halo_l2 | 0 | Reifler & Jones, Efficient Cislunar Multi-Target Tracking with Adaptive Multi-Fidelity Propagation, AMOS 2024, https://amostech.com/TechnicalPapers/2024/Cislunar_SDA/Reifler.pdf |
| C21 | 130 | 24 | 2 | 20.4 | halo_l2 | 0 | Reifler & Jones, AMOS 2024, https://amostech.com/TechnicalPapers/2024/Cislunar_SDA/Reifler.pdf |
| C22 | 46 | 72 | 3 | 20.4 | halo_l2 | 1 | Reifler & Jones, AMOS 2024, https://amostech.com/TechnicalPapers/2024/Cislunar_SDA/Reifler.pdf |
| C23 | 255 | 6 | 1 | 19.7 | nrho | 0 | Givens et al., Novel Algorithms for Custody Maintenance and Tracking in the Cislunar Environment, AMOS 2025, https://amostech.com/TechnicalPapers/2025/Poster/Givens.pdf |
| C24 | 82 | 30 | 2 | 19.7 | nrho | 1 | Givens et al., AMOS 2025, https://amostech.com/TechnicalPapers/2025/Poster/Givens.pdf |
| C25 | 28 | 96 | 3 | 19.7 | nrho | 1 | Givens et al., AMOS 2025, https://amostech.com/TechnicalPapers/2025/Poster/Givens.pdf |
| C26 | 195 | 10 | 1 | 19.3 | halo_l2 | 0 | Evans et al., Visibility Informed Covariance Analysis for Cislunar Periodic Orbits, AMOS 2025, https://amostech.com/TechnicalPapers/2025/Cislunar_SDA/Evans.pdf |
| C27 | 58 | 48 | 2 | 19.3 | halo_l2 | 0 | Evans et al., AMOS 2025, https://amostech.com/TechnicalPapers/2025/Cislunar_SDA/Evans.pdf |
| C28 | 170 | 12 | 2 | 18.8 | nrho | 0 | Tucker et al., Agile Space Object Custody for Electro-Optical Sensors, AMOS 2021, https://amostech.com/TechnicalPapers/2021/Poster/Tucker.pdf |
| C29 | 55 | 48 | 3 | 18.8 | nrho | 1 | Tucker et al., AMOS 2021, https://amostech.com/TechnicalPapers/2021/Poster/Tucker.pdf |
| C30 | 88 | 24 | 2 | 18.5 | nrho | 1 | Gambarotto et al., Monitoring Cislunar fragmentation events using a constellation of space-based observers, Acta Astronautica vol 238 (2025), doi:10.1016/j.actaastro.2025.09.078 |

### Variable construction

| Variable | Definition | Construction / source |
| --- | --- | --- |
| position_3sigma_km | End-of-arc three-sigma position uncertainty (kilometers) reported by the source paper’s orbit-determination filter | Read from covariance summary tables and trace-plots in cited AMOS and Acta Astronautica papers |
| arc_hours | Length of the observation arc used by the OD filter, in hours | Sensor tasking and OD-campaign timelines in source papers |
| n_sensors | Count of distinct contributing sensors (ground sites and space-based observers) during the arc | Sensor-network tables in source papers |
| apparent_mag | Reported apparent visual magnitude of the target during the arc | Photometric statistics in source papers |
| space_based | 1 if at least one space-based observer contributed measurements during the arc, 0 if ground-only | Coded from observer descriptions in source papers |
| orbit_family | Cislunar orbit family: nrho, halo_l1, halo_l2, dro, or lunar_transfer | Identified by source papers |
| source | Bibliographic source for the row, with author, venue, year, and URL or DOI | Manual transcription from the cited paper |

### Estimator output

| Term | Coef. | Std. err. | t | p | 95% CI |
| --- | --- | --- | --- | --- | --- |
| const | -385.5572 | 360.151 | -1.0705 | 0.2844 | [-1091.4401, 320.3257] |
| arc_hours | -0.1302 | 0.4775 | -0.2727 | 0.7851 | [-1.0662, 0.8057] |
| n_sensors | -106.6356 | 30.6081 | -3.4839 | 0.0004942 | [-166.6264, -46.6447] |
| apparent_mag | 38.5749 | 19.979 | 1.9308 | 0.0535 | [-0.5833, 77.733] |
| space_based | -21.8654 | 31.0543 | -0.7041 | 0.4814 | [-82.7307, 38.9999] |

Fit: N = 30 R2 = 0.6882 R2_adj = 0.6383 F = 12.4593 F_p = 1.041e-05