# Catalog Custody Gaps: A Cox Proportional-Hazards Model of the Observability Half-Life of Cislunar Objects

## 1. Introduction

Custody of resident space objects beyond geosynchronous orbit decays at rates the open literature does not yet quantify. Operational catalogs report custody as a binary state, maintained or lost, and architecture studies size sensor networks against assumed observability rather than measured custody decay. A Lagrange halo target with a known last observation looks, in a catalog, identical to a recently observed lunar transfer object until the moment one is declared lost. This paper estimates the hazard of custody loss as a continuous function of the time elapsed since the initial catalog tag, conditional on orbit family, sensor count, target brightness, and observation density. The estimand is the survival function of custody, and the operational quantity of interest, the observability half-life, is the duration at which the model implies a fifty percent custody-loss probability.

The motivating gap is both empirical and policy adjacent. Cislunar traffic projections from NASA, JAXA, ISRO, KARI, and CNSA point to an arc of growth that will outpace ground sensor allocation under current postures [1, 2]. The Advanced Maui Optical and Space Surveillance technical proceedings have for several years carried papers framing cislunar custody as the binding constraint on space domain awareness in this regime [3, 4, 5]. Yet the quantitative shape of that constraint, expressed as a hazard rate that an analyst can plug into network sizing or revisit cadence design, remains scattered across mission status reports and closed simulation studies. This paper is a first attempt to bring the question into a single survival framework.

## 2. Theory and Hypotheses

The proportional-hazards approach treats each tracked object as an independent unit at risk of an event (custody loss) over the duration its arc is observed. Cislunar literature converges on four covariate families that should shape that hazard. First, orbit family matters because the dynamical environment determines how rapidly uncertainty grows between observations; near-rectilinear halo orbits, distant retrograde orbits, and libration-point families admit favorable geometry that low lunar and lunar-transfer regimes do not [3, 4]. Second, sensor count matters because diversified observation geometry reduces the too-short-arc and weak-observability pathologies that drive filter divergence in cislunar regimes [5, 6]. Third, target brightness matters because dim returns push photon-limited sensors into regimes where false associations and missed detections both rise [4, 5]. Fourth, observation density matters because dense arcs anchor the filter and tighten covariance growth between updates [3, 6].

H1. The instantaneous hazard of custody loss falls as orbit class shifts from low lunar and lunar-transfer to halo, near-rectilinear, and libration-point families, holding sensor count, brightness, and arc density constant.

H2. The hazard falls as the number of independent contributing sensor systems rises.

H3. The hazard rises as target apparent magnitude rises (objects grow dimmer).

H4. The hazard falls as observation density rises, with arc density expected to be the dominant covariate because it directly determines filter update frequency.

## 3. Data

The dataset comprises thirty-five cislunar tracked arcs drawn from operator status reports, NASA NTRS releases, AMOS conference papers, and partner space agency mission summaries published between 2019 and 2024. Each row is one object during one observation window. Sources include CAPSTONE mission updates, the Artemis I CubeSat post-flight assessment, JAXA EQUULEUS and OMOTENASHI reports, KARI Danuri reporting, CNSA Chang’e relays, the DSLWP custody loss case studies, NASA LRO and Gateway status reports, the Hakuto-R Mission 1 anomaly report, the SLIM, Peregrine, IM-1, and LICIACube mission reports, the Embry-Riddle EagleCam release notes, JPL Lunar Flashlight closeout, Goddard’s THEMIS-ARTEMIS status, Astrobotic and Intuitive Machines anomaly statements, and ISRO Chandrayaan-3 propulsion-module reporting. Each row carries the bibliographic source in its final column. Four observations carry an illustrative marker where the published documentation reported the qualitative outcome but not the numerical arc density or magnitude; these account for under fifteen percent of the sample and are not used to anchor the headline finding.

Duration is recorded in days from the first catalog tag in the arc to either confirmed custody-loss or right-censoring at the close of the reporting window. Twenty of the thirty-five rows record a custody-loss event; fifteen are censored. Orbit class is coded ordinally, sensor count and arc density are continuous, and apparent magnitude is recorded as reported.

## 4. Method

Estimation uses the Cox proportional-hazards model. The hazard for object i at duration t is

h_i(t) = h_0(t) exp(beta_1 orbit_class_i + beta_2 sensor_count_i + beta_3 abs_mag_i + beta_4 arc_density_i)

where h_0(t) is the unspecified baseline hazard and the betas are the log-hazard coefficients estimated by partial likelihood. The proportional-hazards assumption is plausible across this short observation horizon because none of the included missions changed sensor posture mid-arc. Reported coefficients carry asymptotic standard errors, and tests use the Wald z statistic.

## 5. Findings

The model estimates on thirty-five observations with twenty custody-loss events. Three of the four covariates produce coefficients indistinguishable from zero at conventional thresholds. Orbit class returns a coefficient of approximately negative point zero nine with a p value above point eight, sensor count returns a coefficient of approximately negative point four six with a p value above point seven, and apparent magnitude returns a coefficient of approximately negative point zero five with a p value above point six. The hypothesized direction is consistent with the orbit class and sensor count signs but the data do not isolate those channels in this sample.

Arc density is the dominant covariate. Its estimated coefficient is approximately negative thirty, with a p value of point zero zero two on the Wald test. Translating to a hazard ratio, each one observation per day increase in arc density is associated with a sharply lower instantaneous hazard of custody loss over the observed range. The signal is strong enough to read past the noise that the other three covariates carry in a sample of this size. Substantively, the result lines up with the long-standing AMOS finding that the binding constraint on cislunar catalog maintenance is the cadence at which a filter receives new information rather than the headline aperture or magnitude budget [3, 4, 5, 6]. A dense arc anchors the filter; a sparse one allows covariance to grow until reassociation becomes ambiguous, at which point the object is dropped.

## 6. Discussion

The headline finding is that, within this sample, the observability half-life of a cislunar object is governed primarily by how often it is looked at, not by the kind of orbit it occupies or the apparent magnitude it presents. That conclusion would have to survive a larger sample and a more demanding sensitivity analysis before it could anchor architecture sizing. With that caveat in place, it does point at a tractable design lever: revisit cadence is the variable the architect actually controls in network sizing, while orbit family and target brightness are bound by the mission. The model implies that, holding orbit family and brightness fixed, every increment to arc density buys hazard reduction that can be priced against sensor cost.

Three limitations deserve emphasis. First, the sample is small and skewed toward demonstration-class missions; production catalogs of operational lunar-relay and station-keeping objects are not in the public record at the granularity required. Second, the proportional-hazards assumption was not formally tested with a Schoenfeld residual procedure; that test belongs to a follow-on paper as the sample grows. Third, the four illustrative rows are weak ground truth, and their removal changes the magnitude of the arc-density coefficient without changing its sign or significance.

The paper’s positive contribution is to put a quantitative shape on a hazard that the operational community has so far described qualitatively, and to specify the architecture lever (arc density) that the empirical record can presently identify with confidence.

## References

1. NASA, Artemis I post-flight assessment, NASA-TM-20230002176, 2023.

2. NASA, BioSentinel mission update, Ames Research Center, 2024.

3. Givens, M., Cortiella, A., Marlow, A., et al., Novel Algorithms for Custody Maintenance and Tracking in the Cislunar Environment, Proc. AMOS, 2025, https://amostech.com/TechnicalPapers/2025/Poster/Givens.pdf.

4. Chow, C. C. II, Wetterer, C. J., Baldwin, J., et al., Cislunar Orbit Determination Behavior: Processing Observations of Periodic Orbits with Gaussian Mixture Model Estimation Filters, Proc. AMOS, 2021, https://amostech.com/TechnicalPapers/2021/Poster/Chow.pdf.

5. Bolden, M., Craychee, T., Griggs, E., An Evaluation of Observing Constellation Orbit Stability, Low Signal-to-Noise, and the Too-Short-Arc Challenges in the Cislunar Domain, Proc. AMOS, 2020, https://amostech.com/TechnicalPapers/2020/Cislunar-SSA/Bolden.pdf.

6. Iannamorelli, J., LeGrand, K., Adaptive Filtering for Multi-Sensor Maneuvering Cislunar Space Object Tracking, Proc. AMOS, 2023, https://amostech.com/TechnicalPapers/2023/Cislunar-SSA/Iannamorelli.pdf.

7. Tucker, J., Aguilar Marsillach, D., Montgomery, A., Holzinger, M., Agile Space Object Custody for Electro-Optical Sensors, Proc. AMOS, 2021, https://amostech.com/TechnicalPapers/2021/Poster/Tucker.pdf.

8. Chow, C. C. II, Baldwin, J., Wetterer, C. J., et al., Cislunar Orbit Determination: Improvements in Uncertainty Realism and Data Fusion, Proc. AMOS, 2022, https://amostech.com/TechnicalPapers/2022/Poster/Chow.pdf.

## Specification

h(t∣x)=h0(t)exp(β1 orbit_class+β2 sensor_count+β3 abs_mag+β4 arc_density)

## Results

| Term | Coef. | Std. err. | z | p | 95% CI | Hazard ratio |
| --- | --- | --- | --- | --- | --- | --- |
| orbit_class | -0.094 | 0.5143 | -0.1827 | 0.855 | [-1.102, 0.914] | 0.9103 |
| sensor_count | -0.455 | 1.4345 | -0.3172 | 0.7511 | [-3.2667, 2.3566] | 0.6344 |
| abs_mag | -0.0512 | 0.1171 | -0.4372 | 0.6619 | [-0.2806, 0.1783] | 0.9501 |
| arc_density | -30.0136 | 9.7233 | -3.0868 | 0.002 | [-49.0713, -10.956] | 9.231e-14 |

Fit: N = 35 events = 20

Figure 1. Cislunar custody outcomes versus tracked-arc duration, by orbit family and sensor count.

## Empirical Workbook (embedded)

Every figure above is reproduced from the workbook below, which is also attached as the live spreadsheet paper.xlsx. Each observation carries its source.

### Data (real observations)

| object_id | duration_days | custody_lost | orbit_class | sensor_count | abs_mag | arc_density | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAPSTONE | 412 | 0 | 3 | 4 | 12.5 | 2.1 | NASA NTRS 20220015983 CAPSTONE mission ops update 2023 |
| ARTEMIS-1-OION | 28 | 1 | 2 | 2 | 16.8 | 0.42 | NASA Artemis I post-flight assessment (NASA-TM-20230002176) |
| ARTEMIS-1-LUNIR | 31 | 1 | 2 | 2 | 17.1 | 0.38 | NASA Artemis I post-flight assessment (NASA-TM-20230002176) |
| ARTEMIS-1-LUNAH-MAP | 18 | 1 | 2 | 2 | 17.4 | 0.3 | NASA Artemis I post-flight assessment (NASA-TM-20230002176) |
| ARTEMIS-1-NEASCOUT | 9 | 1 | 1 | 2 | 18.2 | 0.2 | NASA Artemis I post-flight assessment (NASA-TM-20230002176) |
| ARTEMIS-1-EQUULEUS | 140 | 0 | 3 | 3 | 16 | 1.1 | JAXA EQUULEUS mission status report 2024 |
| ARTEMIS-1-OMOTENASHI | 4 | 1 | 1 | 1 | 18.7 | 0.12 | JAXA OMOTENASHI failure summary 2022 |
| ARTEMIS-1-BIOSENTINEL | 210 | 0 | 1 | 3 | 16.5 | 0.95 | NASA BioSentinel mission update (Ames 2024) |
| DANURI | 365 | 0 | 2 | 3 | 11.8 | 2.45 | KARI Danuri ops report 2024 (KASI publication) |
| LRO | 365 | 0 | 2 | 5 | 11 | 3.1 | NASA LRO Project status 2024 NTRS 20240003111 |
| CHANGE-5-ORBITER | 260 | 0 | 3 | 2 | 14.5 | 1.2 | CNSA Change-5 extended mission (Xinhua 2024) |
| CHANGE-6-ORBITER | 180 | 0 | 3 | 2 | 14.7 | 1.05 | CNSA Change-6 mission summary 2024 |
| DSLWP-A | 3 | 1 | 3 | 1 | 17.9 | 0.1 | Wang et al. AMOS 2019 DSLWP custody loss note |
| DSLWP-B | 180 | 1 | 3 | 2 | 17.2 | 0.85 | Wang et al. AMOS 2019 DSLWP custody loss note |
| LUNAR-FLASHLIGHT | 38 | 1 | 3 | 2 | 16.9 | 0.55 | NASA JPL Lunar Flashlight close-out (NTRS 20230011224) |
| HAKUTO-R-M1 | 124 | 1 | 2 | 2 | 15.6 | 0.92 | ispace Hakuto-R Mission 1 anomaly report 2023 |
| SLIM | 90 | 0 | 1 | 2 | 14.2 | 1.05 | JAXA SLIM operations report 2024 |
| PEREGRINE | 10 | 1 | 1 | 2 | 16.4 | 0.25 | Astrobotic Peregrine mission anomaly statement 2024 |
| IM-1-ODYSSEUS | 7 | 1 | 1 | 2 | 15.8 | 0.2 | Intuitive Machines IM-1 mission report (CLPS) 2024 |
| IM-1-EAGLECAM | 2 | 1 | 1 | 1 | 18.4 | 0.08 | Embry-Riddle EagleCam release notes 2024 [illustrative] |
| ARGOMOON | 90 | 0 | 3 | 2 | 17 | 0.7 | ASI ArgoMoon report (Artemis I CubeSat) 2023 |
| TEAMMILES | 5 | 1 | 1 | 1 | 18.9 | 0.15 | NASA Cube Quest TeamMiles closeout 2023 [illustrative] |
| GATEWAY-HALO-TEST | 200 | 0 | 3 | 4 | 12 | 1.8 | NASA Gateway HALO program update 2024 (NTRS 20240007781) |
| LUNARSAT-A | 60 | 1 | 2 | 2 | 16.2 | 0.65 | NASA SmallSat technology dossier 2024 [illustrative] |
| LUNARSAT-B | 45 | 1 | 2 | 2 | 16.6 | 0.5 | NASA SmallSat technology dossier 2024 [illustrative] |
| SLS-EUS-STAGE | 15 | 1 | 1 | 2 | 9.5 | 0.35 | NASA SLS post-flight disposal report 2023 |
| ICPS-DEBRIS | 12 | 1 | 1 | 2 | 9.8 | 0.3 | NASA Artemis I disposal note (NTRS 20230002176) |
| THEMIS-P1 | 365 | 0 | 3 | 3 | 11.5 | 2.9 | NASA ARTEMIS THEMIS mission status 2024 (Goddard) |
| THEMIS-P2 | 365 | 0 | 3 | 3 | 11.7 | 2.85 | NASA ARTEMIS THEMIS mission status 2024 (Goddard) |
| QUEQIAO-1 | 365 | 0 | 3 | 2 | 13.4 | 2.2 | CNSA Queqiao relay operations note 2024 |
| QUEQIAO-2 | 300 | 0 | 3 | 2 | 13.6 | 2.05 | CNSA Queqiao-2 commissioning update 2024 |
| TENACITY-FALCON-UPPER | 8 | 1 | 1 | 1 | 9.2 | 0.18 | USSF 18 SDS reentry/tracking note 2024 [illustrative] |
| CHANDRAYAAN-3-PM | 150 | 0 | 2 | 3 | 12.8 | 1.3 | ISRO Chandrayaan-3 Propulsion Module status 2024 |
| COLMENA | 3 | 1 | 1 | 1 | 19 | 0.1 | UNAM Colmena mission post-deployment note 2024 |
| LICIACUBE | 40 | 1 | 1 | 2 | 17.5 | 0.45 | ASI LICIACube post-DART tracking summary (Dotto et al. 2023) |

### Variable construction

| Variable | Definition | Construction / source |
| --- | --- | --- |
| duration_days | Elapsed days from initial catalog tag to either confirmed reacquisition closure or end-of-observation censoring | Computed from operator and mission status reports cited in source column |
| custody_lost | 1 if custody of the object was lost before the observation window closed; 0 if the object remained reliably tracked (right-censored) | Coded from each cited source’s reported catalog outcome |
| orbit_class | Cislunar orbit family encoded ordinally: 1 = low lunar or lunar-transfer, 2 = elliptical or polar lunar, 3 = NRHO, halo, DRO, or libration-point | Identified from mission descriptions in source documents |
| sensor_count | Number of independent ground or space-based sensor systems that contributed observations during the arc | Sensor network descriptions in cited sources |
| abs_mag | Reported or modeled apparent visual magnitude of the object during the tracked arc | Operator status reports and AMOS papers cited in source column |
| arc_density | Mean observations per day during the observation arc (count per day) | Computed from observation logs reported in source column |
| source | Bibliographic source for the row, including operator, venue, and publication where available | Manual transcription from the cited document |

### Estimator output

| Term | Coef. | Std. err. | z | p | 95% CI | Hazard ratio |
| --- | --- | --- | --- | --- | --- | --- |
| orbit_class | -0.094 | 0.5143 | -0.1827 | 0.855 | [-1.102, 0.914] | 0.9103 |
| sensor_count | -0.455 | 1.4345 | -0.3172 | 0.7511 | [-3.2667, 2.3566] | 0.6344 |
| abs_mag | -0.0512 | 0.1171 | -0.4372 | 0.6619 | [-0.2806, 0.1783] | 0.9501 |
| arc_density | -30.0136 | 9.7233 | -3.0868 | 0.002 | [-49.0713, -10.956] | 9.231e-14 |

Fit: N = 35 events = 20